ANGAWATCH ECO-COMMUNITY

A First-Principles, Solar-and-Biogas Powered Community for 2,000 Families

Yatta Plateau, Machakos County, Kenya · 1.17°S, 37.45°E

| RESIDENTS | SOLAR INSTALLED | MONTHLY SAVING | GENERATION |
| --- | --- | --- | --- |
| ~8,090 | 3,360 kWp | KES 35,000/month | 5,734 MWh/year |

Power the Community 2026 · Energy Mentors International

Submission Deadline: April 15, 2026 by 23:59 GMT (2:59am EAT Apr 16) · energymentors.org/participate

SECTION 01 EXECUTIVE SUMMARY

Angawatch — from the Swahili-Maa fusion of anga (sky/atmosphere) and watch (guardian) — is Kenya’s entry for Power the Community 2026. It is a ground-up community for 2,000 families on the Yatta Plateau, Machakos County, 60km from Nairobi. Angawatch answers the competition’s founding question directly: we do not retrofit solar onto existing houses. We build houses around solar.

## What Angawatch Is — And What It Deliberately Is Not

Angawatch is designed for economic optimisation and housing affordability — not off-grid environmental purity. This distinction is deliberate and honest. A nurse earning KES 40,000/month cannot afford the capital cost of a system engineered to function in complete grid isolation. What she can afford is a home where her electricity bill drops from KES 11,500 to KES 930 per month, where her mortgage is KES 33,015, and where her children go to school 400 metres from her front door. Angawatch is a Grid-Interactive Microgrid with Islanding Capability — it connects to the KPLC grid, exports surplus solar for income, imports emergency power during the long rains, and can island-operate during grid outages. This architecture is cheaper, more resilient, and more bankable than a true off-grid system of equivalent size.

## Why Biogas Goes to Schools and Clinics — Not Homes

The community biogas plant produces 773 m³/day of usable gas. This is not enough to cook for 2,000 families — and any document that claims otherwise is mathematically incorrect. The correct figure for 2,000 households would require approximately 3,200 m³/day, more than four times what our system produces. The honest engineering decision: direct all institutional biogas to the Community Health Centre, two schools, and the community market kitchen. These three facilities currently cook on LPG or charcoal. Replacing their cooking fuel with biogas eliminates PM2.5 exposure for 400 children in school kitchens daily and reduces institutional cooking costs by KES 890,000/year. This is a higher-impact use of limited biogas than spreading it thinly across 2,000 homes where LPG SACCO bulk-procurement already delivers a 15–20% cost reduction. Individual homes transition from SACCO LPG to solar induction cooking in Phase 3 (Year 10+) as solar surplus grows. Maximum impact with the resource available — not maximum coverage.

## The 80/20 Principle — Three Decisions Deliver 80% of All Benefit

In the spirit of the competition’s engineering philosophy — most improvement with least input — three decisions account for 80% of Angawatch’s resident benefit:

- DECISION 1 — Biogas system + community LPG SACCO replacing charcoal. The biogas plant (KES 65M) supplies clean gas to the CHC, schools, and community market, eliminating PM2.5 from institutional cooking entirely. Individual households shift from retail LPG (KES 1,560/month) to community SACCO bulk-procured LPG (KES 1,260–1,330/month), saving KES 230–300/month immediately. Phase 3 (Year 10+) transitions homes to solar induction cooking on surplus solar. The biogas system generates 254 MWh/year electricity (with 40-cow commercial dairy). BCR ~2.8 — still the best investment in the project.

- DECISION 2 — CSEB construction from Yatta volcanic laterite. Eliminates mechanical cooling entirely (zero kWh for AC), reduces build cost 35% vs concrete block, employs community labour during construction, and provides lifelong passive thermal comfort. Zero operating cost.

- DECISION 3 — M-Pesa SmartMeter community tariff at KES 9/kWh. Requires KES 2.5M software development. At 150 kWh/month planned household consumption, saves each family KES 2,400/month versus KPLC’s KES 25/kWh (KES 1,350 vs KES 3,750 for same 150 kWh). A Nairobi-equivalent household consuming 460 kWh/month saves KES 7,360/month — the community tariff rate advantage is 64% regardless of consumption level. Citizens buy their own units, control their own consumption, receive net metering credits directly to their mobile wallet — energy governance controlled by residents, not a central utility.

## Key Metrics

| METRIC | VALUE | NOTES |
| --- | --- | --- |
| Total Families | 2,000 | All four unit types |
| Total Residents | ~8,090 | Per unit-type occupancy: 1BR×1.5 + 2BR×4.0 + 3BR×4.5 + 4BR×6.0 — consistent with Section 05 population table (300+4,520+3,150+120=8,090). |
| Site Area | 100–102 ha (247–252 acres) | Yatta Plateau, Machakos County, 1.17°S, 37.45°E. Expanded from 90ha to accommodate G+2 maisonette terraces for 3BR/4BR units. |
| Solar Installed | 3,264 kWp nameplate / 3,360 kWp effective | 7,680 × Jinko JKM425N-54HL4R-BDB Tiger Neo N-Type Bifacial 425W panels. MATH: 7,680 × 425W = 3,264 kWp nameplate DC capacity. Sunny Design reports 3.36 MWp because it applies the 3% bifacial gain factor (rear-side irradiance from reflected ground light): 3,264 × 1.03 = 3,362 kWp ≈ 3,360 kWp effective. The 425W is the correct panel spec — the apparent discrepancy is the bifacial gain, not an error. |
| Annual Solar Generation | 5,480 MWh/year | Sunny Design independently validated: 5,479.85 MWh/year at PR 0.82, 5.14 kWh/m²/day (Nairobi met station 71km). Previous hand calculation: 5,515 MWh — difference 0.6%, confirming design is sound. |
| Biogas Electricity | 254 MWh/year | 40-cow commercial dairy raises total biogas to 773 m³/day. 539 m³/day to genset at same efficiency = 254 MWh/year. |
| Total Generation | 5,734 MWh/year | Solar 5,480 (Sunny Design validated) + Biogas 254. Export surplus: 1,379 MWh/year. |
| Total Community Demand | 4,355 MWh/year | 4,032 subtotal (incl. borehole pumps 60.2 MWh + irrigation 36.5 MWh revised for sloped site) + 323 losses = 4,355 MWh. |
| Annual Export Surplus | 1,379 MWh/year | 5,734 − 4,355 = 1,379 MWh @ KES 12/kWh = KES 16.5M/year |
| KPLC Grid Import | ~22 MWh/year | Long rains only — April stress month |
| Monthly Family Saving (3BR family) | ~KES 35,000 (housing+energy+transport) / ~KES 41,000 (incl. school) | vs. equivalent Nairobi satellite town. Cooking saving reduced: SACCO LPG vs retail LPG. |
| Community Revenue (Year 5+) | KES 182.1M/year cash / KES 79.0M net | Biochar retained on-site (not sold). Digestate as OPEX saving. Fully closed-loop farm. |

## What Makes Angawatch Original

- YattaFarm Solar-Pumped Canal Irrigation — 15,000m³ retention pond at the highest elevation of YattaFarm. A 10kW solar high-lift pump at the canal boundary pumps water uphill into the pond. The full pond then acts as a massive gravity header-tank, distributing water downhill to the entire drip irrigation network by gravity. First community in Kenya to combine a pumped-fill upslope reservoir with solar-powered gravity distribution to a cooperative farm.

- M-Pesa SmartMeter Energy Payments — Residents buy electricity tokens via USSD/WhatsApp using the same M-Pesa infrastructure 56 million Kenyans already use. Unique Kenyan innovation no other competition team can replicate.

- Biochar from Digestate — Solid fraction of biogas digestate pyrolised into biochar for sale to Machakos flower farms. Carbon-negative, zero-waste, additional revenue.

- Battery Swap Station for Bodas — Ampersand/Roam-model swappable 1.85 kWh packs for 50 Roam Air electric motorcycles. 2-minute swap replaces 8-hour charge wait. Already operational model in Kenya.

- CSEB from Yatta Basaltic Laterite — Yatta Formation volcanic laterite requires only 5% cement stabiliser vs standard 8-10%, reducing embodied carbon and cost.

SECTION 02 SITE SELECTION — YATTA PLATEAU, MACHAKOS COUNTY

Angawatch is sited at 1.17°S, 37.45°E on the Yatta Plateau escarpment, near Matuu town, Yatta Sub-County — 60km southeast of Nairobi along the A109 highway and 25km from Athi River SGR station.

## Site Parameters

| PARAMETER | VALUE / NOTES |
| --- | --- |
| Coordinates | 1.17°S, 37.45°E — near Matuu, Yatta Sub-County, Machakos County |
| Site Area | 100–102 hectares (247–252 acres) — community footprint. Mixed typology: 5-storey apartments (1BR/2BR) + G+2 maisonette terraces (3BR/4BR) requires ~10–12ha more than original 90ha apartment-only design. |
| Elevation | 1,450–1,550m ASL — eliminates need for mechanical cooling (17–27°C year-round) |
| Solar Irradiance | 5.18 kWh/m²/day annual average (PVGIS v5.2 verified, 12-month) |
| Yatta Canal | ~2km from site — gravity irrigation off-take feasible. Canal unsuitable for power generation (flow too weak and inconsistent for turbine). Used for irrigation ONLY. |
| Distance to Nairobi | ~60km via A109; SGR Athi River station 25km |
| KPLC Substation | Existing 33kV substation Matuu ~4km — KPLC grid interconnect KES 36M |
| Rainfall | 700–900mm bimodal: long rains Mar–May, short rains Oct–Dec |
| Soil Type | Basaltic laterite (Yatta Formation) — CSEB-suitable, KS 02-1070 compliant, 5% cement binder only |
| Thwake Dam | 50km southeast — 20MW now operational 2026. Stabilises regional grid. Reduces KPLC blackout risk. |

## Land Use — 100–102 Hectares (Revised for Mixed Typology)

| ZONE | AREA (HA) | % | PURPOSE |
| --- | --- | --- | --- |
| Residential — HYBRID TENURE | 37 | 36% | 1BR apartments (150 units): BUILD-TO-RENT — retained as cooperative rental stock at KES 10,000–12,000/month. 2BR apartments (1,130 units): sold via KMRC Sectional Title mortgages at KES 33,015/month EMI. G+2 maisonette terraces (3BR×700 + 4BR×20 = 720 units): sold via traditional KMRC mortgages. The cooperative retains only 150 units as permanent rental — all other units generate sales proceeds that retire the DFI construction bond. |
| YattaFarm + chrysanthemum plot | 20 | 22% | Canal-irrigated food cooperative, 4ha flowers |
| Community Hub (CHC, schools, worship, recreation) | 12 | 13% | Health, education, worship, sports |
| Jua Kali Industrial Zone | 8 | 9% | Workshops, CSEB factory, EV depot, boda swap |
| Energy Infrastructure (hubs, biogas plant) | 3 | 3% | 4 energy hubs + biogas plant + stormwater pond |
| Roads, paths, EV carports | 7 | 8% | Internal roads + solar carport canopies |
| Green space, food forest, laundry courts | 10 | 11% | Play areas, indigenous trees, cluster courtyards |
| Commercial strip + market | 5 | 6% | Shops, offices, open market, pharmacy |
| TOTAL | 102 | 100% | Site area updated for G+2 maisonette footprint requirement |

SECTION 03 CRITICAL NEEDS — STATEMENT SUPPORTED BY REFERENCES

Every design decision in Angawatch traces to a specific, referenced problem. The following six needs are documented from government, WHO, and field data sources. References appear in Section 22.

## Need 1 — Household Air Pollution from Solid Fuel Cooking (and why LPG alone is not enough)

The World Health Organisation (2022) reports that household air pollution from cooking on solid fuels — charcoal, firewood, animal dung — is responsible for 3.2 million deaths per year globally, including over 237,000 deaths of children under five. Around 2.4 billion people cook this way. In Machakos County, over 70% of rural households cook on charcoal or firewood (KNBS 2019 Census). Indoor PM2.5 concentrations during charcoal cooking reach 300–1,000 µg/m³ — up to 67× the WHO 24-hour safe limit of 15 µg/m³. Angawatch’s three-phase approach: Phase 1 — all residents switch from charcoal to LPG via community bulk SACCO procurement (immediate PM2.5 reduction). Phase 2 — biogas supplied to CHC, schools, and community market (institutional cooking goes near-zero PM2.5). Phase 3 — solar induction cooking for homes (Year 10+ as solar surplus grows). LPG with proper ventilation is a dramatic improvement over charcoal — PM2.5 drops from 300–1,000 µg/m³ to 10–30 µg/m³, far below charcoal but higher than biogas. This phased approach is honest and achievable.

| POLLUTANT | CURRENT YATTA (CHARCOAL) | ANGAWATCH PHASE 1 (LPG HOME) / PHASE 2 (BIOGAS INSTITUTIONS) | WHO SAFE LIMIT |
| --- | --- | --- | --- |
| PM2.5 during cooking (charcoal) | 300–1,000 µg/m³ | 10–30 µg/m³ (LPG Phase 1) / < 10 µg/m³ (CHC/schools biogas Phase 2) / ~0 µg/m³ (solar induction Phase 3) | 15 µg/m³ (annual), 45 µg/m³ (24-hr) |
| Carbon monoxide (CO) | 50–200 ppm | < 5 ppm | 10 ppm (8-hour) |
| NOx | 20–40 ppm | < 5 ppm | No specific WHO limit — lower is always better |
| PM2.5 per household per year | 6.57 kg (charcoal) | ~0.05 kg (LPG Phase 1) / ~0 kg (solar induction Phase 3) | — |
| Community PM2.5 eliminated (vs charcoal baseline) | — | ~13.0 tonnes/year (LPG vs charcoal: 6.57−0.05 kg × 2,000 homes = 13,040 kg) | — |

## Need 2 — Energy Poverty

KPLC connection rates in Yatta sub-county: 45–60%, far below Nairobi’s 95%+. Where KPLC reaches homes, domestic tariffs of KES 25–28/kWh consume a disproportionate share of income. The Yatta region experiences 15–25 power outages per year, with each outage averaging 4–8 hours (KPLC Outage Report 2024). Angawatch’s community solar provides power at KES 9/kWh — 64% cheaper — with 99.6% annual uptime.

## Need 3 — Cooking Cost and Transition Strategy

Current situation: 70%+ of Yatta households cook on charcoal or firewood. The nurse-teacher family that has already switched to LPG pays KES 1,560/month (1.2 × 6kg cylinders). Those still on charcoal pay KES 2,500–4,000/month but suffer severe indoor air pollution. Angawatch’s phased cooking strategy: Phase 1 (Years 1–5) — Community LPG SACCO buys cylinders in bulk from TOTAL Energies Kenya at 15–20% discount. Resident cost: KES 1,260–1,330/month. Saving vs retail LPG: KES 230–300/month. Saving vs charcoal: KES 1,200–2,700/month. Phase 2 — Biogas from the community plant supplies the CHC, schools, and community market kitchen. Phase 3 (Year 10+) — Solar induction cooking for homes as solar surplus grows. From charcoal to LPG to solar: a realistic, funded, 10-year cooking transition.

## Need 4 — Transport Poverty

Current matatu fares: Matuu to Machakos KES 120–180 one way. Return: KES 240–360/day × 22 working days = KES 5,280–7,920/month. Angawatch electric community e-matatu: KES 80–100/trip × 2 × 22 days = KES 3,520–4,400/month. Saving: ~KES 4,840/month. Boda operators currently spend KES 6,000–8,000/month on petrol. Roam Air electric boda: KES 50/day charging = KES 1,500/month. Saving: KES 5,500–6,500/month.

## Need 5 — Healthcare Access

Machakos County doctor-to-patient ratio: 1:8,000 (Machakos County CIDP 2023–2027). WHO recommended minimum: 1:1,000. Angawatch’s 50-bed CHC staffed primarily by resident nurse families brings healthcare within walking distance. Telemedicine link to Machakos Level 5 Hospital via broadband connectivity.

## Need 6 — Education Infrastructure

Yatta sub-county school pupil-to-classroom ratio: 50+ (national average). Angawatch Primary and Secondary schools achieve 30 pupils per class. With approximately 300 resident teacher families, teachers live within walking distance of their workplace — eliminating the teacher housing shortage that afflicts rural Kenyan schools.

SECTION 04 COMMUNITY ARCHETYPES — WHO ANGAWATCH IS BUILT FOR

To guide our engineering parameters, financial modelling, and unit mix decisions, we developed three Core User Archetypes. These are Human-Centered Design composite personas — not specific individuals quoted verbatim, but representative profiles constructed from KNBS 2019 Census data, Kenya TSC salary scales (2023), NCK nurse grade salary schedules, and brief on-the-ground surveys with residents of Matuu and Yatta townships in early 2026. Every design decision — housing size, transport cost, cooking fuel, energy budget — is calibrated against at least one archetype.

## Archetype 1: The Rural Nurse — Composite profile representing public health and education households earning KES 95,000–134,000 combined

This archetype represents the dual-income professional family — the primary beneficiary and most common household type at Angawatch, making up approximately 60% of the 3BR resident population.

| ATTRIBUTE | AGGREGATE DATA (KNBS + SALARY SCALES) |
| --- | --- |
| Demographic profile | Dual-income couple, mid-career. Two school-age children. Occasional elderly parent visiting or co-residing. |
| Income range | KES 95,000–134,000/month combined. Nurse (NCK RN, 5–10yrs): KES 55,000–75,000. Teacher (TSC D2–D3): KES 40,000–62,000. Design validation point: KES 110,000 combined. |
| Current housing (Matuu area) | 3-room rental: KES 12,000–18,000/month. No piped hot water. Shared compound. 15–25 KPLC outages/year. |
| Current electricity cost | KPLC prepaid: KES 6,000–8,000/month (KNBS 2019: Machakos peri-urban average KES 7,200). |
| Current cooking cost | LPG 6kg cylinder KES 1,300–1,500, lasts 21–28 days = KES 1,560/month average (EPRA December 2023 pricing). |
| Current commute | Boda + matatu: KES 250–350/day per person = KES 5,500–7,700/month combined. |
| Debt-to-income (3BR KMRC) | EMI KES 40,803 ÷ net income KES 84,230 = 48.4%. High. AHP subsidy or combined income of KES 136,000+ required for comfortable affordability. |

- What residents in this income bracket actually say when asked about energy costs: ‘Tokens zinaisha haraka sana — na wakati wa load shedding, chakula inaharibika.’ (Tokens run out too fast — and when there’s load shedding, the food goes bad.) This reflects the KNBS finding that 68% of peri-urban Machakos households cite electricity cost and reliability as their top household expense concern after rent.

- What they say about cooking: ‘Smoke from the jiko makes the children cough, but gas is too expensive to buy all at once — you cannot buy half a cylinder.’ This captures the real barrier: not the monthly cost but the upfront refill cost of KES 1,300–1,500 in a single transaction. The SACCO bulk procurement model solves this by spreading the cost.

| ENGINEERING RESPONSE | ANGAWATCH SOLUTION |
| --- | --- |
| Electricity cost | Solar at KES 9/kWh vs KPLC KES 25/kWh. Monthly bill drops from KES 7,200 to KES 930 for same consumption. |
| Reliability | 3,600 kWh BESS + biogas genset = 99.6% annual uptime. ‘Tokens run out’ replaced by ’buy via *384# in 15 seconds.’ |
| Cooking cost + upfront barrier | SACCO bulk LPG at KES 1,260–1,330/month. No large upfront refill cost — billed monthly through cooperative. |
| Commute | E-matatu KES 2,200/month vs KES 6,600 matatu/boda. Walk to CHC and school eliminates two-thirds of commute entirely. |
| Debt-to-income stress | AHP subsidy reduces 3BR EMI by KES 5,000–8,000. 2BR at KES 33,015 EMI is the comfortable entry point at KES 110,000 household income. |

## Archetype 2: The Boda Boda Operator — Composite profile representing transport and informal trade workers earning KES 30,000–48,000/month

This archetype represents the self-employed informal sector worker — the 1BR cooperative rental resident. Approximately 7.5% of the Angawatch population. The honest design tension: this income bracket is at the lower edge of affordability even for the 1BR cooperative rental.

| ATTRIBUTE | AGGREGATE DATA (KNBS + FIELD SURVEYS) |
| --- | --- |
| Demographic profile | Young adult, 25–32. Single or recently married. One or two dependents (younger siblings in school). |
| Income range | KES 30,000–48,000/month from combined informal sources. Boda boda: KES 600–900/day gross = KES 18,000–27,000/month net. Phone repair / small trade supplement: KES 8,000–15,000/month. |
| Current housing | Single room: KES 2,500–5,000/month. Shared latrine. No electricity connection — uses KPLC token vendor. |
| Current fuel cost | Petrol: KES 5,500–8,000/month. Oil + tyres: KES 2,000/month. Total boda running cost: KES 7,500–10,000/month. |
| Current electricity | Token vendor at KES 28–35/kWh (above-rate due to shared metering). KES 400–800/month. |
| Debt-to-income (1BR KMRC) | EMI KES 20,145 ÷ KES 38,000 net = 53%. Too high for sole income. This is the honest design limitation. |

- What residents in this income bracket actually say: ‘Boda ni kazi ngumu — petrol inapanda kila wiki, lakini hakuna alternative.’ (Boda is hard work — petrol rises every week, but there is no alternative.) This reflects EPRA data showing petrol prices in Machakos County rose 34% between January 2022 and December 2023.

- What they say about housing aspiration: ‘Nataka mahali salama — si lazima kubwa, lakini salama na stima inayofanya kazi.’ (I want somewhere safe — not necessarily big, but safe and with electricity that works.) The 1BR cooperative rental at KES 10,000–12,000/month directly addresses this.

| ENGINEERING RESPONSE | ANGAWATCH SOLUTION |
| --- | --- |
| Affordability ceiling | 1BR cooperative rental at KES 10,000–12,000/month — well within range. KMRC mortgage not required. |
| Transport cost | Roam Air electric boda: KES 50/swap × 365 days = KES 18,250/year vs petrol KES 6,600–8,000/month = KES 79,200–96,000/year. Annual saving: KES 61,000–78,000. |
| Income stability | Jua Kali bay for phone repair provides second income stream. Community broadband enables online spare parts ordering (Jumia, Kilimall). |
| Honest limitation | At KES 35,000/month sole income, the KMRC mortgage for any unit is unaffordable without a co-borrower. Cooperative rental is the correct product for this archetype. Ownership pathway opens when income grows or a partner co-borrows. |

## Archetype 3: The Deputy Principal — Composite profile representing senior civil servants and upper-tier professionals earning KES 120,000–200,000 combined

This archetype represents the 4BR resident — approximately 1% of the population. A small but important cohort: if Angawatch cannot attract senior professionals, the schools and CHC cannot function at the standard the community requires.

| ATTRIBUTE | AGGREGATE DATA (TSC + MOH SALARY SCALES) |
| --- | --- |
| Demographic profile | Established family or multi-generational household. 38–50 years old. 3+ children including teenagers. Elderly parent co-residing. |
| Income range | KES 120,000–200,000/month combined or single high income. Deputy Principal (TSC D5): KES 80,000–110,000. Senior Nurse / Clinical Officer (MOH J): KES 65,000–90,000. NGO coordinator: KES 90,000–140,000. |
| Current housing | Renting in Machakos town or Nairobi satellite: KES 30,000–55,000/month for equivalent 4BR space. |
| Current commute | Machakos → Yatta Sub-County: KES 10,000–15,000/month. Or living 2 hours from Nairobi family. |
| Debt-to-income (4BR KMRC) | EMI KES 48,615 ÷ KES 118,000 net combined = 41.2%. Comfortable range. Does not require AHP subsidy. |

- What residents at this income level say about Yatta: ‘The land is good and it is not far, but who will teach my children if all the good teachers are in Nairobi?’ This is the circular problem Angawatch breaks: senior professionals will not move to areas that lack quality schools and medical facilities. The integrated 30-pupil-per-class school and 50-bed CHC are the answer.

- What they say about energy: ‘I work night shifts — I need the lights and the fridge to work. I cannot budget for a generator every time KPLC fails.’ The 99.6% uptime argument is more important for this archetype than the cost savings.

| ENGINEERING RESPONSE | ANGAWATCH SOLUTION |
| --- | --- |
| Talent attraction | 4BR unit with private garden creates the equivalent of a professional estate. The Deputy Principal who can walk to work and whose children are in a 30-student class will not commute from Machakos town. |
| Energy reliability | BESS + biogas = 99.6% uptime. Cold chain for home medical storage uninterrupted. This archetype is most likely to have home medical equipment (insulin, respiratory). |
| Unsold unit fallback | Any of the 20 × 4BR units not sold on KMRC mortgage can be rented at KES 42,000–45,000/month to NGO coordinators on 6–12 month postings or locum medical officers. At full rental: 20 × KES 43,500 × 12 = KES 10.4M/year additional cooperative income. |

SECTION 05 HOUSING — WHY NURSES AND TEACHERS CHOOSE ANGAWATCH

The most important question the competition judges will ask is: would real nurses and teachers actually want to live here? The answer must be yes for reasons that go beyond cost savings — people do not move their families 60km from Nairobi for a spreadsheet. These are the genuine pull factors.

## 5.1 Pull Factors — Why Angawatch, Not Just ‘****Somewhere Cheaper****’

| PULL FACTOR | WHAT IT MEANS IN PRACTICE | WHO IT MATTERS MOST TO |
| --- | --- | --- |
| Walk to work | Nurse Wanjiku works at the Angawatch CHC, not at Machakos Level 5 Hospital 38km away. Teacher Kamau teaches at Angawatch Secondary, not at a school 20km away. The daily 2–3 hour commute and KES 6,600+/month transport cost simply disappears. | Every resident. The commute burden is the single biggest quality-of-life drain for rural Kenyan professionals. |
| Ownership, not tenancy | A KMRC mortgage builds equity toward a full title deed. After 25 years Wanjiku’s family owns property on the Yatta Plateau, which is appreciating as infrastructure improves. This is generational wealth — not paying KES 18,000/month to a landlord forever with nothing to show for it. | Families with children. Every Kenyan professional aspires to own land. |
| Children’s schools — walk, not commute | 30 pupils per class vs 50+ in sub-county average. Labs, internet, computers, teachers who live on-site. Kamau’s children attend the same school where Kamau teaches. No school transport cost. No dark morning walks. | Parents with school-age children — the majority of 3BR residents. |
| Professional community | A nurse living among 300 other nurse families is surrounded by professional peers, continuing education opportunities, and a support network. Same for teachers. This is the equivalent of a professional residential estate — not an anonymous apartment block. | Mid-career professionals who value peer networks. |
| No energy stress | KPLC outages kill food in the fridge, leave children doing homework by candle, interrupt medical equipment at home. Angawatch’s 99.6% annual uptime eliminates energy anxiety entirely. The lights always work. The fridge always runs. | Every household. Especially those with children studying or medical equipment. |
| Indoor air quality for children | Charcoal smoke causes chronic respiratory illness in children. Switching from charcoal to LPG (Phase 1) immediately reduces indoor PM2.5 from 300–1,000 µg/m³ to 10–30 µg/m³. Wanjiku already knows this as a nurse — her children’s persistent cough has a cause. | Parents of young children. Medical professionals who understand the data. |
| Net metering credit to M-Pesa | Every month, the community solar system earns money for the resident — a net metering credit arrives on their phone, unprompted. This is not rent, not a bill — it is income from living in the right place. | All residents. The psychological impact of ‘my home pays me’ is significant. |
| Security | Gated Ekeja clusters with single entry. Nurse Wanjiku on a night shift knows her family is in a secure compound. No random strangers walking through the compound at 2am. | Families where one parent works shifts (nurses especially). |

## 5.2 Rent vs Own — The Dual Tenure Model

Not every resident will be ready or able to take on a 25-year mortgage from Day 1. Angawatch operates a dual tenure model: the community cooperative owns a block of units that are rented to residents who are in the mortgage qualification process, on short-term contracts, or simply prefer renting. This also provides early cash flow for the cooperative during the construction phase.

| TENURE TYPE | MONTHLY COST | QUALIFICATION | NOTES |
| --- | --- | --- | --- |
| SACCO Rental (1BR only — 150 units) | KES 10,000–12,000/month | Any employed resident. No mortgage needed. | The ONLY cooperative rental tier. All 2BR and 3BR units are SOLD, not rented. This rental tier is reserved for the 1BR studio only. |
| KMRC Mortgage (2BR) | KES 33,015/month EMI (VAT-adjusted) | KES 110,000+ combined income. 10% deposit (KES 393,750). | 25-year. Builds equity. Full title deed at end. All 1,130 × 2BR units sold via this route. |
| KMRC Mortgage (3BR — ownership) | KES 40,803/month EMI (VAT-adjusted) | KES 136,000+ combined income without AHP. 10% deposit (KES 486,750). | 25-year. 600 units sold via this route. |
| Cooperative rental (3BR) | 100 3BR units | Managed by community cooperative. Mid-level families needing 3BR space while saving deposit. | KES 30,000/month × 100 × 12 = KES 36M/year cooperative income. |
| Cooperative ownership (4BR) | 20 4BR units | Community cooperative manages waitlist for very senior staff. | Unsold 4BR units rented at KES 42,000/month as additional cooperative income. |

The rental option means Angawatch is accessible to a junior nurse (KES 40,000 sole income) who cannot qualify for a mortgage today but wants to establish roots in the community. Rental income from cooperative-owned units is already included in the KES 178.4M/year community revenue figure.

## 5.3 Population Verification — Aligned with Competition Brief

| UNIT TYPE | COUNT | OCCUPANCY | RESIDENTS | BASIS |
| --- | --- | --- | --- | --- |
| 1BR (42m²) — 1BR rental | 150 | 2.0 per unit | 300 | Singles and childless couples — ‘minus’ demographic |
| 2BR (75m²) — CORE FAMILY UNIT | 1,130 | 4.0 per unit (parents + 2 children sharing) | 4,520 | Competition brief baseline: 2 parents + 2 children = 4 persons. 2BR is the Kenyan Starter Family Home. |
| 3BR (95m²) — CORE FAMILY UNIT | 700 | 4.5 per unit (family grown — eldest needs own room) | 3,150 | Same family at a later life stage: 2 parents + 2 children needing separate rooms + occasional elderly parent. |
| 4BR (115m²) — multi-generational | 20 | 6.0 per unit | 120 | Larger or multi-generational families — ‘plus’ demographic |
| TOTAL | 2,000 | Avg 4.05 | 8,090 | Hits the ~8,000 resident target precisely. 91.5% of units (1,830) are Core Family Units. |

Population arithmetic: 300 + 4,520 + 3,150 + 120 = 8,090. This is the cleanest possible answer to the competition‘s ’family of 4, plus or minus’ brief: 1,830 units designed exactly for families of 4; 170 units for the plus/minus cases.

## 5.4 Honest Income Range

ALIGNED WITH COMPETITION BRIEF: The competition specifies a baseline demographic of a 4-person family (2 parents, 2 children). The Angawatch unit mix was reverse-engineered from this constraint. The 2-Bedroom apartment is the quintessential Kenyan Starter Family Home: parents in the master bedroom, two children sharing the second bedroom — a 4-person household in a 2BR. The 3-Bedroom maisonette is for the same family when children are older and need separate rooms. Together, 1,130 × 2BR + 700 × 3BR = 1,830 units = 91.5% of the entire community. These are the ‘Core Family Units.’ The remaining 8.5%: 150 × 1BR for young professionals and childless couples (the ‘minus’ demographic), and 20 × 4BR for larger multi-generational families (the ‘plus’ demographic). This mix literally quotes the competition brief’s own ‘plus or minus’ language back to the judges.

| PROFESSIONAL | SALARY RANGE (GROSS) | NOTES |
| --- | --- | --- |
| Registered Nurse (KNR, degree) | KES 40,000–75,000 | Junior = KES 40K. Experienced = KES 55-65K. Senior = KES 75K. |
| Secondary School Teacher (TSC) | KES 37,000–77,000 | D1 entry = KES 37K. D2 experienced = KES 45-55K. D3 senior = KES 65-77K. |
| Junior-mid combined (5-10yr) | KES 87,000–110,000 | Primary design validation point. Most residents will be here. |
| Senior combined (10yr+) | KES 120,000–140,000 | Upper bracket. Minority of residents. |

## KMRC Affordability — VAT-Inclusive Construction Cost KES 46,500/m² (All units sold except 150 × 1BR cooperative rentals)

EMI Formula: P × [r(1+r)^n] / [(1+r)^n − 1] where r = 0.7492%/month, n = 300 months. Factor = 0.008384.

Construction cost KES 46,500/m² (VAT-inclusive blended): KES 43,000/m² base + 16% VAT on non-exempt materials. Solar, CSEB, batteries are VAT-exempt (Finance Act 2023). Non-exempt materials (cement, steel, cable, fittings) attract 16% VAT. Blended rate ~8% increase over ex-VAT base. Still 25–28% below equivalent concrete block construction at VAT-inclusive rates.

| UNIT | AREA | BUILD COST (46,500/m²) | LAND+INFRA | TOTAL (VAT-INC) | EMI @ 8.99% | % KES 87K GROSS | % KES 110K GROSS | VERDICT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1BR | 42m² | KES 1,953,000 | KES 450,000 | KES 2,403,000 | KES 20,145 | 23.2% ✓ | 18.3% ✓ | Affordable at all incomes |
| 2BR | 75m² | KES 3,487,500 | KES 450,000 | KES 3,937,500 | KES 33,015 | 37.9% — AHP needed | 30.0% — just within KMRC | Needs KES 110K+ combined |
| 3BR | 95m² | KES 4,417,500 | KES 450,000 | KES 4,867,500 | KES 40,803 | 46.9% ✗ | 37.1% ✗ — AHP essential | Needs KES 136K+ gross / AHP subsidy |
| 4BR | 115m² | KES 5,347,500 | KES 450,000 | KES 5,797,500 | KES 48,615 | 55.9% impossible | 44.2% ✗ | Senior staff ONLY (KES 162K+) |

The 3BR at 37.1% of KES 110K gross exceeds KMRC 30% guideline. AHP subsidy is not optional — it is required for any mid-level household buying 3BR. The AHP subsidy can reduce effective EMI by KES 5,000–10,000/month via interest subsidy and capital grant. Without AHP, the 3BR is only accessible at KES 136,000+ combined gross income. The 2BR at KES 33,015 (30% of KES 110K) is the genuinely accessible ownership unit for the typical mid-level nurse-teacher family.

## Unit Mix — 2BR and 3BR Dominant

| TYPE | COUNT | % | EMI (VAT-ADJ) | MIN GROSS INCOME | TARGET |
| --- | --- | --- | --- | --- | --- |
| 1BR (42m²) — ownership | 150 | 7.5% | KES 20,145 | KES 67,000 | Singles, childless couples. Mohamed: SACCO rental first. |
| 2BR (75m²) — ownership | 1,130 | 56.5% | KES 33,015 | KES 110,000 | PRIMARY OWNERSHIP. Mid-level families. At 30% of KES 110K — exactly at KMRC ceiling. |
| 3BR (95m²) — ownership | 600 | 30% | KES 40,803 | KES 136,000 or AHP subsidy | Senior-experienced staff. AHP subsidy essential for lower earners. |
| 3BR (95m²) — cooperative rental | 100 | 5% | KES 30,000/month | KES 100,000 | Mid-level families needing 3BR space while saving deposit. Priority purchase right in 3–5 years. |
| 4BR (115m²) — ownership | 20 | 1% | KES 48,615 | KES 162,000 | Absolute minimum. Realistic senior-grade pool at Angawatch: ~30–35 Deputy Principal teachers + ~20 Chief Nurses = ~50–70 total who CAN afford it, but most prefer 3BR with lower EMI. 20 units is honest. |
| TOTAL | 2,000 | 100% | Avg KES 32,400 | Avg ~KES 109K | 2BR+3BR = 91.5%. 4BR reduced to 1% — 20 units only. |

## Hybrid Tenure Model — Apartments Rent, Maisonettes Own

Angawatch employs a Hybrid Tenure Model that matches the financial reality of its two target demographics. The architecture already splits into two typologies — this section formalises the corresponding financial model for each.

| TYPOLOGY | UNITS | TENURE MODEL | MONTHLY COST | RATIONALE |
| --- | --- | --- | --- | --- |
| 1BR studio apartments (42m²) | 150 units | BUILD-TO-RENT — cooperative retains ownership | KES 10,000–12,000/month | These are the ONLY cooperative rental units. A manageable portfolio. Revenue: 150 × KES 11,000 × 12 = KES 19.8M/year. No billion-shilling debt needed. Entry barrier = zero deposit. |
| 2BR apartments (75m²) — Core Family Units | 1,130 units | SECTIONAL TITLE MORTGAGE — sold to residents via KMRC 8.99% + ~1.5% MRIS (Mortgage Reducing Insurance Scheme) = effective ~10.45% p.a. | EMI KES 33,015/month (inclusive of MRIS) | Starter Family Home for the competition’s baseline 4-person family. Parents + 2 children in 75m². Sold, not rented: families of 4 want equity and ownership, not perpetual tenancy. KMRC provides the mortgage. Cooperative recovers construction cost from sales receipts. |
| 3BR maisonette terraces (95m²) — Core Family Units | 700 units — ALL SOLD via KMRC mortgage | KMRC MORTGAGE (all 700 units sold) | EMI KES 40,803 | All 700 × 3BR units sold to established families seeking ownership. Previously, 100 were incorrectly listed as cooperative rental — this was removed when we eliminated the cooperative rent spiral risk. All 700 are now correctly in the sales column. |
| 4BR maisonette terraces (115m²) | 20 units | KMRC MORTGAGE — sold to senior staff | EMI KES 48,615/month | Very senior staff only. Genuine ownership by families who can afford it. |

WHY ONLY 150 1BR UNITS ARE RENTED: Build-to-rent for all 1,280 apartments would require the cooperative to borrow ~KES 4.48B at 7–9% DFI rates while achieving only a 5.4% gross rental yield — a financial death spiral. The correct model: 1,130 × 2BR sold via KMRC Sectional Title Mortgages; 720 maisonettes sold via KMRC mortgages. Only the 150 × 1BR studios are retained as cooperative rental (KES 19.8M/year). KMRC provides mortgages to end-user homebuyers — not to cooperative landlords. All 1,850 sold units generate KES 7,974M that retires the DFI construction bond (4,449+3,409+116 = 7,974M).

## Ekeja Cluster Model — Apartment Organisation

Angawatch organises its 1,280 apartment units into 20 Ekeja clusters of approximately 64 units each, arranged around a shared central courtyard containing a laundry court, children’s play area, reed-bed greywater garden, and community seating. Each cluster has its own CATL EnerOne BESS sub-unit and microgrid capable of islanding independently.

## Construction Features — Every Unit

- CSEB walls: Yatta basaltic laterite + 5% cement binder, pressed on-site by Angawatch Block Enterprise. Lower stabiliser ratio than standard 8-10% due to natural volcanic mineral binders in Yatta Formation rock.

- Yatta laterite-lime render: 15mm external render using on-site laterite + 15% hydrated lime + 5% CSEB fines. Breathable, moisture-regulating, 60% solar reflectance. Not cement plaster — not concrete block aesthetics.

- LPG connection stub in kitchen: 2-burner hob connection point. Phase 1: community SACCO-procured LPG cylinders at bulk discount rate (KES 1,260–1,330/month). Phase 3: converted to solar induction cooker as surplus solar grows. Biogas supplies CHC, schools, community market — NOT individual apartments (insufficient yield from human waste alone).

- M-Pesa SmartMeter (Kaifa MA105): Token-based prepayment. Residents buy units via USSD *384# or WhatsApp. Tokens SMS-delivered in <15 seconds. Net metering credits returned to M-Pesa wallet.

- Solar water heater (flat-plate 2m² per unit): Eliminates electric geyser load. Delivers 100–120 litres of hot water daily at Yatta’s 5.18 kWh/m²/day irradiance.

- Standard adjustable aluminium louvred windows with externally mounted insect screens and internal lever operation: NOT double-glazed (wrong climate — Yatta is warm, not cold). Louvres allow precise control of ventilation — open during cool SE trade wind breezes, closed during dust storms. External screens keep insects out without blocking lever access. Essential for malaria-endemic Machakos County.

- Ventilated roof cavity: 150mm air gap between concrete slab and corrugated steel roof sheet. Ridge vents at roof apex allow convective escape of hot air. Keeps slab temperature 8–12°C below roof surface temperature.

- No mechanical air conditioning: CSEB thermal mass + ventilated cavity + louvred windows maintain 20–27°C interior at 1,450–1,550m elevation. Heating demand = ZERO (minimum ambient 12°C in July — above WHO comfort minimum of 18°C for sleeping rooms, comfortable in Kenyan cultural context).

- Broadband fibre (FTTB): Safaricom or Jamii Telecom (Faiba) fibre to building riser. Cat6 horizontal run to each apartment. Essential for teachers (lesson prep, KNEC marking), nurses (NHIF portal, telemedicine), and Mohamed’s online spare parts ordering.

- Communal laundry courtyard per Ekeja cluster: Concrete line-posts, covered drying section, floor drainage. 4 × semi-commercial 8kg front-load washing machines per cluster (1 per 25 units). No tumble dryers — sun-drying is standard Kenyan practice.

SECTION 06 FLOOR PLANS &** ELECTRICAL LOAD SCHEDULES — NYUMBA YA ANGAWATCH**

All four unit types have been drawn and verified in Floorplanner.com. The actual drawn dimensions are recorded below alongside the design specification. Where minor variances exist (1BR: 46m² drawn vs 42m² spec; 2BR: 78m² vs 75m²; 3BR: 101m² vs 95m²; 4BR: 117m² vs 115m²) these reflect wall thickness inclusion in the drawing tool and are within normal architectural tolerance. Each unit type is accompanied by a full electrical load schedule — the basis for the energy audit in Section 8.

## DUAL TYPOLOGY — How Units Sit in Their Buildings

1BR and 2BR units (1,280 total) occupy 5-storey Ekeja apartment blocks. Ground floor: 2 commercial units. Floors 2–5: 4 apartments per floor around a central stairwell and lift core. 3BR and 4BR units (720 total) are single-storey bungalow maisonettes with private front gardens (4m deep), private entrances at ground level, and rear drying yards. The typology split matches the tenure model: apartments are cooperative rental, maisonettes are KMRC mortgage sales.

## UNIT 1 — 1-Bedroom Studio

As-Drawn Dimensions: 7.30m × 6.30m | Floor Plan Area: 46m² | Spec: 42m²

| ROOM | DRAWN SIZE | AREA | KEY FEATURES | ENERGY/UTILITY |
| --- | --- | --- | --- | --- |
| Living / sleeping (open plan) | 5.39m × 3.34m | 18m² | Sofa bed. Convertible layout. Louvred windows east + west faces for cross-ventilation. Ceiling fan provision. | LED 9W × 3. Fan circuit. |
| Kitchen alcove | 2.60m × 2.07m | 5.4m² | Biogas 2-burner hob. Under-counter fridge space. Window above sink. | Biogas pipeline + meter |
| Bathroom + WC combined | 2.27m × 1.54m | 3.5m² | Low-flow shower. WC. Basin. Solar hot water feed pipe from roof collector. | Solar thermal pipe |
| Entrance hall + meters | 1.26m × 1.29m | 1.6m² | Kaifa MA105 SmartMeter. LPG meter. Shoe rack. Coat hooks. | Meter cabinet |
| Balcony | 1.94m × 1.87m | 3.6m² | East-facing. Planting trough. Retractable clothesline. Louvred privacy screen. | Natural ventilation |
| Hallway / circulation | 1.26m × 1.54m | 1.9m² | Connects entrance to bathroom and living area. |  |
| TOTAL (drawn) |  | ~34m² internal + walls | Wall thickness accounts for difference from 42m² spec. Acceptable. |  |

1BR Electrical Load Schedule — 2 Residents

| APPLIANCE | QTY | WATTS EACH | TOTAL W | HRS/DAY | DAILY kWh | MONTHLY kWh | NOTES |
| --- | --- | --- | --- | --- | --- | --- | --- |
| LED lights (9W) | 4 | 9W | 36W | 5.0 | 0.180 | 5.4 | Living, kitchen, bathroom, entrance |
| Ceiling fan | 1 | 50W | 50W | 4.0 | 0.200 | 6.0 | Peak heat season Apr, Sep–Jan |
| Refrigerator 150L | 1 | 60W avg | 60W | 24 | 0.432 | 13.0 | 30% duty cycle = 18W avg drawn |
| LED TV 32-inch | 1 | 50W | 50W | 3.0 | 0.150 | 4.5 | Evening viewing |
| Phone charging × 2 | 2 | 10W | 20W | 2.0 | 0.040 | 1.2 | USB-C fast charge |
| Laptop | 1 | 45W | 45W | 3.0 | 0.135 | 4.1 | Work / study |
| Electric kettle 1.8kW | 1 | 1,800W | 1,800W | 0.25 | 0.450 | 13.5 | 3 boils × 5 min — chai culture |
| Clothes iron | 1 | 1,000W | 1,000W | 0.10 | 0.100 | 3.0 | 6 min/day average |
| WiFi router | 1 | 10W | 10W | 24 | 0.240 | 7.2 | Always on |
| Security motion light | 1 | 15W | 15W | 2.0 | 0.030 | 0.9 | Entrance/balcony sensor |
| USB charging hub 4-port | 1 | 20W | 20W | 3.0 | 0.060 | 1.8 | Multiple devices |
| Blender (occasional) | 1 | 300W | 300W | 0.05 | 0.015 | 0.5 | Food prep |
| Miscellaneous (radio, clock) | — | — | 15W avg | 4.0 | 0.060 | 1.8 | Small devices |
| SUBTOTAL RAW |  |  |  |  | 2.092 kWh | 62.8 kWh |  |
| + 15% system losses |  |  |  |  | 0.314 | 9.4 | Wiring + inverter + BESS |
| TOTAL WITH LOSSES |  |  |  |  | 2.406 kWh/day | 72.2 kWh/month | Annual × 150 units = 131.8 MWh/year |

Peak simultaneous load (kettle + iron + TV + fan + lights): 1,800+1,000+50+50+36 = 2,936W ≈ 3.0kW. Circuit breaker: 16A.

## UNIT 2 — 2-Bedroom Apartment

As-Drawn Dimensions: 10.30m × 7.60m | Floor Plan Area: 78m² | Spec: 75m²

FLOOR PLAN CORRECTIONS REQUIRED: (1) Kitchen drawn at 2.60m × 2.07m = 5.4m² — needs to be minimum 3.0m wide for LPG hob, oven point and worktop. Redraw kitchen to 3.0m × 3.0m. (2) Living room at 2.59m × 3.34m = 8.7m² is too small for a family of 4. A family sofa arrangement needs minimum 4.0m × 3.5m. Extend living room by borrowing from hallway width — reduce hallway to strict 1.2m corridor. (3) The large master bedroom at 5.42m × 3.02m = 16.4m² is actually oversized for a 2BR — reduce to 4.0m × 3.5m and give the saved space to the living room.

| ROOM | DRAWN SIZE | AREA | KEY FEATURES | ENERGY/UTILITY |
| --- | --- | --- | --- | --- |
| Living / dining (needs enlarging) | 2.59m × 3.34m (current) → 5.0m × 3.5m (corrected) | 17.5m² | South + east facing. Louvred windows both faces. Ceiling fan. Dining table for 4. L-shaped sofa. | LED 9W × 4. Fan circuit. |
| Kitchen (needs widening) | 2.60m × 2.07m (current) → 3.0m × 3.0m (corrected) | 9m² | Biogas 2-burner + 4-burner upgrade point. Full worktop. Solar water pipe. Window above sink. | Biogas + meter |
| Master bedroom | 5.42m × 3.02m | 16.4m² | East-facing. Teacher study desk 1.2m × 0.6m at window. Built-in wardrobe. Louvred window. | LED 9W × 2 |
| Bedroom 2 | 5.0m × 2.49m | 12.5m² | North or west facing. Single bed. Built-in wardrobe. Child’s study desk. | LED 9W × 1 |
| Bathroom | 1.54m × 3.34m | 5.1m² | Solar hot water. Low-flow shower. Basin. WC (combined). Mirror. | Solar thermal |
| Separate toilet | 0.80m × 1.30m | 1.0m² | Near living room. WC + hand basin. |  |
| Hallway + meters | 1.15m wide corridor | ~5m² | Kaifa MA105 + LPG meter. Linen cupboard. Connects all rooms. | Meter cabinet |
| TOTAL (drawn) |  | ~78m² | 3m² over spec — acceptable within wall thickness tolerance. |  |

2BR Electrical Load Schedule — 4 Residents (Parents + 2 Children)

| APPLIANCE | QTY | WATTS EACH | TOTAL W | HRS/DAY | DAILY kWh | MONTHLY kWh | NOTES |
| --- | --- | --- | --- | --- | --- | --- | --- |
| LED lights (9W) | 7 | 9W | 63W | 5.0 | 0.315 | 9.5 | Living × 3, bedrooms × 2, kitchen, bathroom |
| Ceiling fans | 2 | 50W | 100W | 5.0 | 0.500 | 15.0 | Living room + master bedroom |
| Refrigerator 200L | 1 | 85W avg | 85W | 24 | 0.612 | 18.4 | 30% duty cycle |
| LED TV 40-inch | 1 | 65W | 65W | 4.0 | 0.260 | 7.8 | Family evening viewing |
| Phone charging × 4 | 4 | 10W | 40W | 2.0 | 0.080 | 2.4 | Parents + older children |
| Laptop × 1 | 1 | 45W | 45W | 3.0 | 0.135 | 4.1 | Teacher lesson prep / nurse portal |
| Electric kettle 1.8kW | 1 | 1,800W | 1,800W | 0.30 | 0.540 | 16.2 | 4 boils/day — larger family |
| Clothes iron | 1 | 1,000W | 1,000W | 0.15 | 0.150 | 4.5 | Daily uniform pressing |
| WiFi router | 1 | 10W | 10W | 24 | 0.240 | 7.2 | Always on — KNEC marking, NHIF portal |
| Security motion light | 1 | 15W | 15W | 2.0 | 0.030 | 0.9 | Entrance sensor |
| USB charging hub 4-port | 1 | 20W | 20W | 4.0 | 0.080 | 2.4 | Multiple devices overnight |
| Blender | 1 | 300W | 300W | 0.08 | 0.024 | 0.7 | Food prep |
| Children study lamp × 2 | 2 | 9W | 18W | 3.0 | 0.054 | 1.6 | Homework — LED desk lamps |
| Blood pressure monitor (nurse) | 1 | 5W | 5W | 0.10 | 0.001 | 0.0 | Medical professional equipment |
| Miscellaneous (radio, nightlight, clock) | — | — | 20W avg | 4.0 | 0.080 | 2.4 |  |
| SUBTOTAL RAW |  |  |  |  | 3.101 kWh | 93.0 kWh |  |
| + 15% system losses |  |  |  |  | 0.465 | 14.0 |  |
| TOTAL WITH LOSSES |  |  |  |  | 3.566 kWh/day | 107.0 kWh/month | Annual × 1,130 units = 1,472 MWh/year |

Peak simultaneous load (kettle + iron + fans × 2 + TV + lights): 1,800+1,000+100+65+63 = 3,028W ≈ 3.0kW. Circuit breaker: 20A (4.8kW capacity).

## UNIT 3 — 3-Bedroom Maisonette

As-Drawn Dimensions: 9.80m × 10.30m | Floor Plan Area: 101m² | Spec: 95m²

FLOOR PLAN CORRECTIONS REQUIRED: (1) Three bedrooms drawn at similar sizes — differentiate: master should be largest (15m²), bedroom 2 medium (12m²), bedroom 3 smallest (10m²). (2) En-suite bathroom missing from master bedroom — the 3BR spec explicitly requires a 5m² en-suite attached to master. Add a 2.5m × 2.0m en-suite partition accessed from master bedroom only. (3) Overall 101m² is 6m² over spec — trimming the hallway width and slightly reducing bedroom 1 will bring this to ~95m².

| ROOM | DRAWN SIZE | AREA | KEY FEATURES | ENERGY/UTILITY |
| --- | --- | --- | --- | --- |
| Living / dining | 4.97m × 4.92m | 24.5m² | East-south corner. Exposed CSEB feature wall (leave bricks visible on longest wall). Louvred windows east + west. 2 ceiling fans. Sofa + dining table for 6. | LED 9W × 4. 2 fans. |
| Kitchen | 2.87m × 2.98m | 8.6m² | Biogas 4-burner hob + oven point. Full worktop. Sink at window looking rear. Ventilation stack corner. Wall cabinets above. | Biogas + meter |
| Master bedroom | 3.92m × 3.72m | 14.6m² | South-east. En-suite access. Teacher study desk 1.2m × 0.6m at east window. Built-in wardrobe full wall. King bed. | LED 9W × 3 |
| Master en-suite (ADD) | 2.5m × 2.0m | 5.0m² | Walk-in shower. WC. Vanity basin. Solar hot water feed. Small louvred window. | Solar thermal |
| Bedroom 2 — older child | 3.72m × 3.42m | 12.7m² | East-facing. Queen bed. Built-in desk at window for homework. Wardrobe 1.8m wide. | LED 9W × 2 |
| Bedroom 3 — younger child / study | 2.77m × 3.42m | 9.5m² | North-facing (quietest). Murphy bed. Built-in desk. Wardrobe. Doubles as study room. | LED 9W × 2 |
| Family bathroom | 2.77m × 1.01m (expand to 2.5m × 2.0m) | 5.0m² | Full bathtub + overhead shower. WC. Basin. Solar hot water. Shared bedrooms 2 + 3. | Solar thermal |
| Hallway + storage + meters | 1.60m × 4.92m | 7.9m² | Kaifa + LPG meters. Large linen cupboard. Wide hallway connecting all rooms. | Meter cabinet |
| TOTAL (drawn + corrections) |  | ~95m² after adjustments | En-suite addition + bedroom resizing brings to spec. |  |

3BR Electrical Load Schedule — 4–5 Residents (Established Family)

| APPLIANCE | QTY | WATTS EACH | TOTAL W | HRS/DAY | DAILY kWh | MONTHLY kWh | NOTES |
| --- | --- | --- | --- | --- | --- | --- | --- |
| LED lights (9W) | 10 | 9W | 90W | 5.0 | 0.450 | 13.5 | Living × 4, master × 2, bed2, bed3, en-suite, family bath |
| Ceiling fans | 2 | 50W | 100W | 5.0 | 0.500 | 15.0 | Living room + master bedroom |
| Refrigerator 250L | 1 | 90W avg | 90W | 24 | 0.720 | 21.6 | Larger family needs larger fridge. 30% duty. |
| LED TV 43-inch (living) | 1 | 75W | 75W | 4.0 | 0.300 | 9.0 | Family viewing. Teacher uses for KNEC marking. |
| Phone charging × 5 | 5 | 10W | 50W | 2.0 | 0.100 | 3.0 | Parents + 3 children (older ones have phones) |
| Laptop × 2 | 2 | 45W | 90W | 4.0 | 0.360 | 10.8 | Teacher lesson prep + student homework |
| Electric kettle 1.8kW | 1 | 1,800W | 1,800W | 0.35 | 0.630 | 18.9 | Larger family, more chai |
| Clothes iron | 1 | 1,000W | 1,000W | 0.15 | 0.150 | 4.5 | School uniforms daily |
| WiFi router | 1 | 10W | 10W | 24 | 0.240 | 7.2 | Always on |
| Security motion light × 2 | 2 | 15W | 30W | 3.0 | 0.090 | 2.7 | Front garden + rear yard |
| USB charging hub 6-port | 1 | 30W | 30W | 4.0 | 0.120 | 3.6 | All family devices |
| Blender / food processor | 1 | 400W | 400W | 0.10 | 0.040 | 1.2 | Cooking prep for larger family |
| Children study lamp × 3 | 3 | 9W | 27W | 3.0 | 0.081 | 2.4 | One per child studying |
| Medical equipment — nurse | 1 | 10W | 10W | 0.20 | 0.002 | 0.1 | BP monitor, thermometer, pulse ox |
| Printer — teacher | 1 | 50W | 50W | 0.20 | 0.010 | 0.3 | KNEC marking, lesson plans |
| Miscellaneous (radio, nightlights, clocks) | — | — | 25W avg | 5.0 | 0.125 | 3.8 |  |
| SUBTOTAL RAW |  |  |  |  | 3.918 kWh | 117.5 kWh |  |
| + 15% system losses |  |  |  |  | 0.588 | 17.6 |  |
| TOTAL WITH LOSSES |  |  |  |  | 4.506 kWh/day | 135.2 kWh/month | Annual × 700 units = 1,154 MWh/year |

Peak simultaneous load (kettle + iron + fans × 2 + TV + lights): 1,800+1,000+100+75+90 = 3,065W ≈ 3.1kW. Circuit breaker: 20A. This unit has the most intensive load profile due to dual professional household (teacher + nurse) with multiple study-age children.

## UNIT 4 — 4-Bedroom Bungalow

As-Drawn Dimensions: 11.39m × 10.30m | Floor Plan Area: 117m² | Spec: 115m²

FLOOR PLAN ASSESSMENT: The 4BR plan is the strongest of the four. The living room at 6.97m × 3.98m = 27.7m² is excellent — generous enough for a full family arrangement. The kitchen at 4.42m × 3.0m = 13.3m² is well sized. Two bathrooms are correct (en-suite to master + family bathroom). The smallest bedroom at 2.67m × 3.67m = 9.8m² is tight but workable for the youngest child with a Murphy bed. The narrow bathroom strip at 0.98m wide needs to be expanded to minimum 1.5m — a person cannot turn around in 0.98m. The 2.3m² overage on spec is minor and acceptable.

| ROOM | DRAWN SIZE | AREA | KEY FEATURES | ENERGY/UTILITY |
| --- | --- | --- | --- | --- |
| Living / dining (open plan to kitchen) | 6.97m × 3.98m | 27.7m² | South-east corner. Open plan. 2 ceiling fans. Sofa for 6 + dining table for 8. TV unit. Breakfast bar dividing kitchen zone. | LED 9W × 5. 2 fans. |
| Kitchen | 4.42m × 3.00m | 13.3m² | Biogas 4-burner + oven. Full worktop. Breakfast bar. Sink + window to rear yard. Wall cabinets. Generous workspace. | Biogas + meter |
| Master bedroom | 4.88m × 3.67m | 17.9m² | South-facing. King bed. Built-in wardrobe full wall. Teacher study desk at window. Access to en-suite. | LED 9W × 3 |
| Master en-suite | 2.67m × 1.83m | 4.9m² | Walk-in shower. WC. Vanity. Solar hot water. Louvred window. | Solar thermal |
| Bedroom 2 — eldest child | 3.67m × 3.67m (approx) | 13.5m² | East-facing. Queen bed. Full-size desk. Built-in wardrobe. Teenager room. | LED 9W × 2 |
| Bedroom 3 — middle child | 2.67m × 4.16m | 11.1m² | North-facing. Single bed. Desk. Wardrobe. | LED 9W × 2 |
| Bedroom 4 — youngest / grandparent | 2.67m × 3.13m | 8.4m² | North-facing. Murphy bed. Shelving. Flexible use room. | LED 9W × 1 |
| Family bathroom (expand from 0.98m) | 2.5m × 2.4m (corrected from 0.98m) | 6.0m² | Full bath + overhead shower. WC. Basin. Solar hot water. Shared bedrooms 2, 3, 4. | Solar thermal |
| Hallway + entrance + storage | Central corridor | 9.5m² | Kaifa + LPG meters. Large storage cupboard. Linen store. Wide entrance lobby with shoe rack. | Meter cabinet |
| TOTAL (drawn) |  | ~117m² | 2m² over spec — minor. Family bathroom width correction needed. |  |

4BR Electrical Load Schedule — 6 Residents (Large/Multi-generational Family)

| APPLIANCE | QTY | WATTS EACH | TOTAL W | HRS/DAY | DAILY kWh | MONTHLY kWh | NOTES |
| --- | --- | --- | --- | --- | --- | --- | --- |
| LED lights (9W) | 14 | 9W | 126W | 5.0 | 0.630 | 18.9 | All rooms: living × 4, kitchen × 2, master × 2, bed2 × 2, bed3, bed4, en-suite, family bath |
| Ceiling fans | 3 | 50W | 150W | 5.0 | 0.750 | 22.5 | Living room × 2 + master |
| Refrigerator 300L | 1 | 105W avg | 105W | 24 | 0.840 | 25.2 | Largest fridge — 6-person household. 30% duty. |
| LED TV 50-inch (living) | 1 | 90W | 90W | 4.0 | 0.360 | 10.8 | Main family TV |
| LED TV 32-inch (master bedroom) | 1 | 50W | 50W | 2.0 | 0.100 | 3.0 | Parents evening viewing |
| Phone charging × 6 | 6 | 10W | 60W | 2.0 | 0.120 | 3.6 | All family members |
| Laptop × 2 | 2 | 45W | 90W | 4.0 | 0.360 | 10.8 | Teacher + student |
| Electric kettle 1.8kW | 1 | 1,800W | 1,800W | 0.40 | 0.720 | 21.6 | Largest household, most chai |
| Clothes iron × 2 | 2 | 1,000W | 2,000W | 0.15 | 0.300 | 9.0 | Two working professionals |
| WiFi router | 1 | 10W | 10W | 24 | 0.240 | 7.2 | Always on |
| Security motion light × 2 | 2 | 15W | 30W | 3.0 | 0.090 | 2.7 | Front garden + rear |
| USB charging hub 8-port | 1 | 40W | 40W | 5.0 | 0.200 | 6.0 | Whole family devices |
| Blender / food processor | 1 | 400W | 400W | 0.12 | 0.048 | 1.4 | Larger family portions |
| Children study lamp × 4 | 4 | 9W | 36W | 3.0 | 0.108 | 3.2 | School-age children |
| Medical equipment — nurse | 1 | 15W | 15W | 0.25 | 0.004 | 0.1 | Professional equipment |
| Printer | 1 | 50W | 50W | 0.25 | 0.013 | 0.4 | Lesson plans, school admin |
| Stereo / music system | 1 | 30W | 30W | 2.0 | 0.060 | 1.8 | Family entertainment |
| Miscellaneous (nightlights, clocks, radio) | — | — | 30W avg | 5.0 | 0.150 | 4.5 |  |
| SUBTOTAL RAW |  |  |  |  | 5.093 kWh | 152.8 kWh |  |
| + 15% system losses |  |  |  |  | 0.764 | 22.9 |  |
| TOTAL WITH LOSSES |  |  |  |  | 5.857 kWh/day | 175.7 kWh/month | Annual × 20 units = 42.8 MWh/year |

Peak simultaneous load (2 × iron + kettle + fans × 3 + TV 50 + lights): 2,000+1,800+150+90+126 = 4,166W ≈ 4.2kW. Circuit breaker: 25A (6.0kW capacity). This is the highest peak load in the community — the 25A breaker provides adequate headroom.

## Load Schedule Summary — All Unit Types

| UNIT TYPE | UNITS | DAILY kWh (with losses) | MONTHLY kWh | ANNUAL MWh (all units) | PEAK LOAD | CIRCUIT BREAKER |
| --- | --- | --- | --- | --- | --- | --- |
| 1BR Studio | 150 | 2.406 | 72.2 | 131.8 | ~3.0 kW | 16A |
| 2BR Apartment | 1,130 | 3.566 | 107.0 | 1,471.6 | ~3.0 kW | 20A |
| 3BR Maisonette | 700 | 4.506 | 135.2 | 1,153.9 | ~3.1 kW | 20A |
| 4BR Bungalow | 20 | 5.857 | 175.7 | 42.8 | ~4.2 kW | 25A |
| Communal laundry (20 clusters × 4 machines) | — | — | — | 79.0 | — | — |
| RESIDENTIAL + LAUNDRY TOTAL | 2,000 | Avg 3.68 | Avg 110.4 | 2,879.1 | ~1,055 kW community peak | — |

IMPORTANT NOTE: The load schedule totals here (2,879 MWh/year) are more detailed than the earlier energy audit (2,724 MWh/year) because additional loads have been incorporated: security lights, USB charging hubs, children study lamps, medical equipment, printers, and stereo systems — items present in real Kenyan professional households but omitted in the initial audit. The Section 8 energy demand total of 4,355 MWh/year includes community facilities (CHC, schools, farm, EV fleet, pumps) and is not affected by this residential refinement. The residential component within Section 8 should be read as 2,726 MWh (audit baseline — corrected); the load schedules here provide the per-unit breakdown.

SECTION 07 PASSIVE DESIGN &** BUILDING ENGINEERING**

All energy efficiency in the housing units is achieved through passive design — no active mechanical systems. At Yatta Plateau’s elevation and latitude, first-principles analysis shows that passive measures alone are sufficient to maintain comfort across all seasons.

## 1. Thermal Mass — CSEB Walls

300mm CSEB walls (Yatta basaltic laterite + 5% cement) provide a thermal mass of approximately 450 kJ/m²·K. Yatta’s diurnal temperature swing is 12–14°C (peak 29–32°C afternoon, trough 16–18°C dawn). The CSEB wall absorbs heat during the hot afternoon and releases it slowly overnight, maintaining interior temperatures within 20–26°C throughout the 24-hour cycle. This eliminates all mechanical cooling — the walls are the air conditioner.

## 2. Ventilated Roof Cavity — NOT Insulation

Wall insulation is NOT specified and would be harmful. Standard insulation (rockwool, foam board) traps heat inside the building at night, preventing the natural cooling that Yatta’s cold nights provide. Instead, a 150mm ventilated air cavity sits between the concrete slab and the corrugated iron roof sheet. Hot air from the roof surface rises through the gap and exits via ridge vents, keeping the slab 8–12°C cooler than the roof surface. This eliminates roof-to-ceiling radiant heat gain during the hottest hours. Heating demand = ZERO. Yatta’s minimum temperature (12°C, July nights) is within the range Kenyan residents find comfortable under bedding — no heating system is installed or budgeted.

## 3. Yatta Laterite-Lime Render — Local Material Innovation

External wall surfaces use Yatta laterite-lime render: 15mm of on-site Yatta Formation basaltic laterite mixed with 15% hydrated lime and 5% CSEB fines. This is not standard cement plaster. It is breathable (vapour-permeable), allowing the CSEB wall to regulate moisture without condensation. The off-white natural pigment achieves 60% solar reflectance vs 30% for dark cement plaster. Applied by community self-help groups trained during construction — lowers labour cost and transfers skills. Local name: ‘Rangi ya Ardhi’ (Earth Colour).

## 4. Cross-Ventilation — Louvred Windows

Standard adjustable aluminium louvred windows — NOT casement (casement = side-hinged swinging frame, incompatible with louvres) and NOT double-glazed (cold climate technology, would trap daytime heat). Each apartment has east-facing and west-facing louvred windows: horizontal aluminium slat louvres operated by an internal hand lever. The lever turns the slat angle from fully closed to fully open, providing 3–5 air changes per hour. Externally mounted insect screens (fine mesh, no frame intrusion into lever mechanism) protect against malaria-bearing mosquitoes. Stack ventilation through the open stairwell draws air passively upward through the building.

## 5. Solar Water Heating — Eliminating Electric Geysers

Every unit has a 2m² flat-plate solar thermal collector on the roof feeding a 150L insulated tank. At 5.18 kWh/m²/day irradiance and 65% thermal efficiency, each system delivers 95–120 litres of hot water per day (sufficient for 4–5 people showering). Electric instant water heaters (Ariston-type commonly used in Kenyan middle-income housing) draw 3–5 kW — their complete elimination saves 1.5 kWh/day per household = approximately 1,095 MWh/year community-wide (2,000 units × 1.5 kWh × 365). This saving is already embedded in the residential appliance audit tables which show 0 kWh for water heating — the solar thermal system is the reason.

## 6. Underground Cabling — Not Overhead

All internal distribution cables are underground XLPE armoured cable. Overhead line alternatives were considered and rejected: (1) Machakos County experiences severe afternoon thunderstorms March–May — overhead lines suffer disproportionate fault rates during these events. (2) A 90-hectare planned community with 20 Ekeja clusters and regular tree planting would require continuous wayleave clearing for overhead lines. (3) Underground cabling is more aesthetically appropriate for a premium community and eliminates visual clutter over green spaces and courtyards.

## Summary — Passive Design Performance Targets

| STRATEGY | TARGET OUTCOME | ENERGY SAVING |
| --- | --- | --- |
| CSEB thermal mass (300mm) | Interior temp 20–26°C year-round | Eliminates 100% of cooling load |
| Ventilated roof cavity (150mm) | Slab 8–12°C below roof surface | Reduces radiant ceiling heat gain 60% |
| Adjustable aluminium louvred windows (NOT double-glazed, NOT casement) | 3–5 ACH natural ventilation | Fan use reduced 70% |
| Yatta laterite-lime render (60% reflectance) | Surface temp 4–6°C below cement plaster | Lowers courtyard ambient 1–2°C |
| Solar water heating (2m²/unit) | 95–120L hot water/day per unit | ~1,095 MWh/year saved community-wide vs electric geysers. Already embedded in appliance audit (geyser = 0 kWh in all unit tables). |
| Heating demand | Zero | No heating system installed |

SECTION 08 TOTAL COMMUNITY ENERGY DEMAND — SECTOR-BY-SECTOR

Complete annual electrical energy demand calculation for all 2,000 units and all community facilities. Cooking is supplied by community SACCO LPG (not electricity). Solar water heating eliminates electric geyser loads. No AC units — passive thermal mass design maintains comfort.

## System Sizing Methodology — Software vs First-Principles

Angawatch uses a two-track methodology to validate its energy system design, avoiding both pure software black-box dependence and pure hand-calculation risk.

- TRACK 1 — Primary Solar PV Generation: Validated using SMA Sunny Design 6.11.2, the industry-standard tool from the world’s largest solar inverter manufacturer. Site coordinates -1.07°S, 37.43°E. Nairobi meteorological station (71km). Validated yield: 5,479.85 MWh/year at 1,630 kWh/kWp specific yield, PR 0.82. String design safety confirmed: max DC voltage 973V safely below 1,000V inverter limit. This is not a hand calculation — it is third-party software validation with a dated project report (April 7, 2026) attached as Appendix A.

- TRACK 2 — BESS Sizing, Dispatch Logic, and Biogas Integration: Calculated via transparent, first-principles engineering documented in full in Section 11. CATL LFP degradation modelled at 6,000-cycle rated life. Overnight demand calculated from the sector-by-sector audit in this section. Biogas dispatch calculated from measured feedstock inputs (verified dairy cow count, digester efficiency factors from IEA Bioenergy). This approach was chosen over HOMER Pro or similar simulation tools because: (1) transparent assumptions can be audited line-by-line by any engineer; (2) CATL-specific degradation curves are embedded in the manufacturer datasheet, not a generic simulation library; (3) the biogas-BESS-grid three-way dispatch logic in our system does not match any standard simulation tool’s preset architecture.

JUDGES NOTE: The Sunny Design report validates the solar yield component (94% of total generation) with instrument-grade precision. The BESS and biogas calculations use conservative engineering assumptions documented in Sections 10 and 11. Any reviewer can audit every number from first principles.

## 8.1 Residential Demand — By Unit Type (Split Tables)

1-Bedroom Units — 150 units — 2 residents average

| APPLIANCE | WATTAGE | DAILY HOURS | DAILY Wh | NOTES |
| --- | --- | --- | --- | --- |
| LED lights × 4 | 36W total | 5.0 hrs | 180 | Living, kitchen, bathroom, entrance |
| Ceiling fan × 1 | 50W | 5.0 hrs | 250 | Hot season April, Sep–Jan |
| Refrigerator 150L | 60W avg | 24 hrs | 432 | 30% duty cycle = 18W avg drawn |
| LED TV 32“ | 50W | 3.0 hrs | 150 | Evening |
| Phone charging × 2 | 10W total | 2.0 hrs | 20 | USB-C |
| Laptop | 45W | 3.0 hrs | 135 | One device |
| Electric kettle 1.8kW | 1,800W | 0.25 hrs | 450 | 3 boils × 5 min — chai culture |
| Clothes iron | 1,000W | 0.10 hrs | 100 | 6 min/day average across week |
| WiFi router | 10W | 24 hrs | 240 | Always on |
| Miscellaneous (radio, USB lamps) | — | — | 120 |  |
| RAW TOTAL |  |  | 2,077 Wh/day |  |
| + 15% system losses |  |  | 311 Wh | Wiring, inverter, BESS round-trip |
| TOTAL WITH LOSSES |  |  | 2,388 Wh/day |  |
| Monthly |  |  | 71.6 kWh | Rounded to 72 kWh/month |
| Annual × 150 units |  |  | 130,743 kWh | ≈ 131 MWh/year (2,388 Wh/day × 365 days × 150 units) |

Peak simultaneous load: kettle + iron + fan + TV + lights = 1,800 + 1,000 + 50 + 50 + 36 = 2,936W ≈ 3.0 kW. Circuit breaker: 16A (3.84 kW capacity).

2-Bedroom Units — 1,130 units — 3–4 residents average

| APPLIANCE | WATTAGE | DAILY HOURS | DAILY Wh | NOTES |
| --- | --- | --- | --- | --- |
| LED lights × 7 | 63W total | 5.0 hrs | 315 | Living, 2 bedrooms, kitchen, bathroom × 2 |
| Ceiling fans × 2 | 100W total | 5.0 hrs | 500 | Living room + master bedroom |
| Refrigerator 200L | 85W avg | 24 hrs | 612 | 30% duty cycle |
| LED TV 40“ | 65W | 4.0 hrs | 260 | Evening family use |
| Phone charging × 4 | 20W total | 2.0 hrs | 40 | Whole family |
| Laptop | 45W | 3.5 hrs | 158 | Work/school laptop |
| Children tablet × 1 | 15W | 2.0 hrs | 30 | Homework |
| Electric kettle 1.8kW | 1,800W | 0.33 hrs | 594 | 4 boils daily |
| Clothes iron | 1,000W | 0.12 hrs | 120 | 7 min/day average |
| Blender (occasional) | 300W | 0.08 hrs | 24 | Weekend use |
| WiFi router | 10W | 24 hrs | 240 | Always on |
| Miscellaneous | — | — | 80 |  |
| RAW TOTAL |  |  | 2,973 Wh/day |  |
| + 15% system losses |  |  | 446 Wh |  |
| TOTAL WITH LOSSES |  |  | 3,419 Wh/day |  |
| Monthly |  |  | 102.6 kWh | Rounded to 103 kWh/month |
| Annual × 1,130 units |  |  | 1,396,680 kWh | ≈ 1,397 MWh/year (103 kWh/month × 12 × 1,130) |

Peak load: 1,800 + 1,000 + 100 + 65 + 63 = 3,028W ≈ 3.0 kW. Circuit breaker: 20A per apartment.

3-Bedroom Units — 700 units (600 ownership + 100 cooperative rental) — 4–5 residents average

| APPLIANCE | WATTAGE | DAILY HOURS | DAILY Wh | NOTES |
| --- | --- | --- | --- | --- |
| LED lights × 9 | 81W total | 5.5 hrs | 446 | 3 bedrooms, living, kitchen, 2 bathrooms, hallway |
| Ceiling fans × 2 | 100W total | 6.0 hrs | 600 | Living + master |
| Refrigerator 250L | 95W avg | 24 hrs | 684 | Larger for 4–5 people, 30% duty |
| LED TV 43“ | 80W | 4.5 hrs | 360 | Family viewing |
| Phone charging × 5 | 25W total | 2.0 hrs | 50 | All family members |
| Teacher laptop (heavy use) | 65W | 5.0 hrs | 325 | Lesson prep, KNEC marking, reports |
| Children tablets × 2 | 30W total | 2.5 hrs | 75 | Two children homework |
| Electric kettle 1.8kW | 1,800W | 0.40 hrs | 720 | 5 boils daily — family of 5 |
| Clothes iron | 1,000W | 0.13 hrs | 130 | 8 min/day average |
| WiFi router | 10W | 24 hrs | 240 | Always on |
| Miscellaneous (radio, USB lamps, shaver) | — | — | 100 |  |
| RAW TOTAL |  |  | 3,730 Wh/day |  |
| + 15% system losses |  |  | 560 Wh |  |
| TOTAL WITH LOSSES |  |  | 4,290 Wh/day |  |
| Monthly |  |  | 128.7 kWh | Rounded to 129 kWh/month |
| Annual × 700 units |  |  | 1,083,600 kWh | ≈ 1,084 MWh/year (129 kWh/month × 12 × 700 = 1,083.6 → 1,084 MWh) |

The teacher laptop running 5 hours/day adds 325 Wh/day vs the 2BR unit. This is specific to the teacher-in-residence: KNEC exam marking, O-level lesson plans, student reports. The laptop is essential professional equipment, not optional.

4-Bedroom Units — 20 units — 5–6 residents average

| APPLIANCE | WATTAGE | DAILY HOURS | DAILY Wh | NOTES |
| --- | --- | --- | --- | --- |
| LED lights × 11 | 99W total | 5.5 hrs | 545 | 4 bedrooms, all common areas |
| Ceiling fans × 2 | 100W total | 6.0 hrs | 600 | Living + master |
| Refrigerator 280L | 110W avg | 24 hrs | 792 | Largest household, 30% duty |
| LED TV 50“ | 100W | 4.5 hrs | 450 | Large family room |
| Phone charging × 6 | 30W total | 2.0 hrs | 60 | Whole family + elderly parent |
| Laptops × 2 | 110W total | 4.0 hrs | 440 | Parent work + eldest child school |
| Electric kettle 1.8kW | 1,800W | 0.45 hrs | 810 | 6 boils daily |
| Clothes iron | 1,000W | 0.15 hrs | 150 | 9 min/day average |
| WiFi router | 10W | 24 hrs | 240 |  |
| Miscellaneous | — | — | 120 |  |
| RAW TOTAL |  |  | 4,207 Wh/day |  |
| + 15% system losses |  |  | 631 Wh |  |
| TOTAL WITH LOSSES |  |  | 4,838 Wh/day |  |
| Monthly |  |  | 145 kWh/month |  |
| Annual × 20 units |  |  | 34,800 kWh | ≈ 35 MWh/year (145 kWh/month × 12 × 20) |

Residential Energy Summary — All Unit Types

| CATEGORY | UNITS | kWh/MONTH | ANNUAL MWh | NOTES |
| --- | --- | --- | --- | --- |
| 1BR units | 150 | 72 | 131 | Corrected: 2,388 Wh/day × 365 × 150 = 130,743 kWh |
| 2BR units | 1,130 | 103 | 1,397 | Count: 1,130. 103 × 12 × 1,130. |
| 3BR units (ownership + rental) | 700 | 129 | 1,084 | Count: 700 (600 own + 100 rental). 129 × 12 × 700 = 1,083.6 → 1,084 MWh. |
| 4BR units | 20 | 145 | 35 | Count: 20 — reduced from 50. Realistic senior-grade pool at Angawatch is ~50 households but most prefer 3BR. 145 × 12 × 20. |
| Communal laundry (20 clusters × 4 machines × 3 cycles × 0.9 kWh × 365) | — | — | 79 | 20 Ekeja clusters, 4 machines each |
| RESIDENTIAL + LAUNDRY TOTAL | 2,000 | Avg 96 | 2,726 | 131 + 1,397 + 1,084 + 35 + 79 = 2,726 MWh. 3BR corrected from 1,082 to 1,084 MWh. |

RESIDENTIAL TOTAL: 2,726 MWh/year. Final corrected figure: (1) 2BR increased to 1,130 units; (2) 4BR reduced to 20 units (from 50 — only ~50–70 households at Angawatch can afford this tier, and most prefer 3BR); (3) 3BR: 700 units (600 ownership + 100 cooperative rental). Average per-unit consumption: 96 kWh/month.

## 8.2 Community Health Centre — 50-Bed Facility

| SYSTEM | DAILY kWh | ANNUAL MWh | PRIORITY |
| --- | --- | --- | --- |
| Ward + corridor LED lighting | 115 | 42 | Standard |
| Theatre + procedure lighting | 84 | 31 | Standard |
| Pharmacy cold chain 2–8°C (CRITICAL) | 72 | 26 | UNINTERRUPTIBLE — dedicated BESS circuit |
| Neonatal incubators × 4 (CRITICAL) | 86 | 31 | UNINTERRUPTIBLE |
| Oxygen concentrators × 4 (CRITICAL) | 25 | 9 | UNINTERRUPTIBLE |
| Laboratory (haematology, biochemistry) | 72 | 26 | Standard |
| Digital DR X-ray unit | 32 | 12 | Standard |
| Patient monitors × 10 (CRITICAL) | 43 | 16 | UNINTERRUPTIBLE |
| Ventilation + admin + security | 96 | 35 | Standard |
| TOTAL CHC | 625 | 228 | — |

## 8.3 Schools — Primary + Secondary

| CONSUMER | ANNUAL MWh | BASIS |
| --- | --- | --- |
| Primary School (20 classrooms) | 10.6 | LED lighting + computers + admin + kitchen |
| Secondary School (30 classrooms + labs) | 27.0 | Labs add significant load — fume hoods, equipment |
| SCHOOLS TOTAL | 37.6 | 250 school days/year |

## 8.4 Jua Kali Industrial Zone — 50 Bays

50 workshop bays: metal fabrication, carpentry, auto mechanics, tailoring, electronics repair. Diversity factor 0.60 (not all bays at full load simultaneously). MANDATORY extraction fans for health and safety.

| LOAD COMPONENT | DAILY kWh/BAY | × 50 BAYS × 0.60 DIVERSITY × 300 DAYS | ANNUAL MWh |
| --- | --- | --- | --- |
| Extraction fans (1.1 kW × 8hr) | 8.80 | — | — |
| Arc welding (5 kW × 3hr intermittent) | 15.00 | — | — |
| Power tools + angle grinder | 5.52 | — | — |
| LED industrial lights + misc | 2.92 | — | — |
| TOTAL PER BAY | 32.24 kWh/day | 50 × 32.24 × 0.60 × 300 | 289,000 |
| Jua Kali annual demand | — | — | 289 MWh (use 302 MWh with 5% misc margin) |

## 8.5 YattaFarm Cold Storage — 2 × 20-Tonne Rooms

| PARAMETER | VALUE |
| --- | --- |
| Refrigeration unit per room | Carrier 38AK 15 kW scroll compressor |
| Ambient temperature (peak Yatta summer) | 32°C |
| Storage temperature | 4°C vegetables / 3°C chrysanthemums |
| Average compressor duty cycle | 65% at peak ambient |
| Average draw (2 rooms combined) | 19.5 kW |
| Daily energy | 468 kWh/day |
| Annual energy | 171 MWh/year (24/7 operation) |

## 8.6 EV Fleet and Boda Swap Station

| VEHICLE | SPEC | DAILY ENERGY | ANNUAL MWh |
| --- | --- | --- | --- |
| 5 × Roam Rapid 14-seat e-matatu | 68 kWh battery, 0.35 kWh/km | 127 kWh/bus × 5 | 232 |
| 50 × Roam Air electric boda (swap station) | 1.85 kWh pack, swapped once/day | 2.04 kWh/boda × 50 | 37 |
| EV TOTAL | — | — | 269 |

## 8.7 Common Services

| SERVICE | ANNUAL MWh |
| --- | --- |
| Street and path lighting (250 × 40W LED × 12hr × 365) | 44 |
| Water treatment plant | 65 |
| Community centre, market, admin offices | 50 |
| Biogas plant auxiliary (pumps, sensors, compressor) | 25 |
| Security cameras + access gates | 15 |
| M-Pesa billing server + IT infrastructure | 5 |
| COMMON SERVICES TOTAL | 204 |

## 8.8 TOTAL COMMUNITY DEMAND SUMMARY

| SECTOR | ANNUAL MWh | % | PEAK kW | SUPPLY PRIORITY |
| --- | --- | --- | --- | --- |
| Residential (all units + laundry) | 2,726 | 61.9% | ~840 | Revised: 1BR×150(131) + 2BR×1,130(1,397) + 3BR×700(1,084) + 4BR×20(35) + laundry(79) = 2,726 MWh. 3BR corrected from 1,082 to 1,084. |
| Community Health Centre | 228 | 5.0% | 52 | CRITICAL — uninterruptible sub-circuit |
| Schools (primary + secondary) | 37.6 | 0.8% | 21 | Standard (school hours only) |
| Jua Kali Industrial Zone | 302 | 6.6% | 216 | Standard (business hours 7am–6pm) |
| YattaFarm Cold Storage | 171 | 3.7% | 20 | Semi-critical (24/7) |
| YattaFarm irrigation high-lift pump (10kW solar, 10hr/day) | 36.5 | 0.8% | 10 | Deferrable (daytime solar hours only). The Yatta Plateau slopes uphill from the canal. A 10kW pump handles ~30m total dynamic head across the 102ha sloped site — lifting water from the canal-level sump to the upslope retention pond and then from the pond to the highest drip zones. Previous 5kW spec assumed a flat 15m head which was insufficient for actual site topography. |
| Borehole pumps — potable water supply (3 × Grundfos SP17-7, 5.5kW, 10hr/day) | 60.2 | 1.3% | 16.5 | Solar-powered 8am–6pm. Lifts 400m³/day from 60m depth. Previously not accounted for. |
| EV Matatu + Boda Swap Station | 269 | 5.8% | 280 | Deferrable (mid-day DC fast charge + overnight BESS) |
| Common services | 204 | 4.4% | 80 | Standard |
| Subtotal before losses | 4,032 | 92.6% |  |  |
| Add 8% network losses | 323 | 7.4% |  |  |
| GRAND TOTAL WITH LOSSES | 4,355 | 100% | ~1,055 kW peak | Verified: 2,724 + 228 + 37.6 + 302 + 171 + 36.5 + 60.2 + 269 + 204 = 4,032 subtotal + 323 losses (8%) = 4,355 MWh. Export surplus: 5,734 − 4,355 = 1,379 MWh/year @ KES 12 = KES 16.5M/year. |

## 8.9 Two-Season Daily Energy Profile

DRY SEASON DAY — July (Best Solar, Biogas Normal)

| TIME PERIOD | SOLAR OUTPUT | BESS STATE | BIOGAS GENSET | COMMUNITY LOAD | KPLC |
| --- | --- | --- | --- | --- | --- |
| 5am–7am | 0 kW | Discharging (500 kW available) | Running 180 kW | ~380 kW residential + CHC | Off |
| 7am–10am | 200–1,400 kW rising | Charging begins 8am | Off (sun up) | ~500 kW (EV fast charge 8:15–9:00) | Off |
| 10am–4pm | ~2,600 kW solar peak | 100% charged by 2pm | Off | ~700 kW all sectors active | Exporting ~1,900 kW surplus |
| 4pm–7pm | 800 kW declining | Discharging from 5pm | Off | ~600 kW (evening peak) | Off |
| 7pm–midnight | 0 kW | Discharging 300 kW | Running 180 kW | ~450 kW (residential eve) | Off |
| Midnight–5am | 0 kW | Discharging 150 kW | Running 180 kW | ~280 kW (CHC + cold storage + minimal residential) | Off |

WET SEASON DAY — April (Lowest Solar, Biogas Critical)

| TIME PERIOD | SOLAR OUTPUT | BESS STATE | BIOGAS GENSET | COMMUNITY LOAD | KPLC |
| --- | --- | --- | --- | --- | --- |
| 5am–7am | 0 kW | Discharging | Running 180 kW | ~380 kW | Off |
| 7am–10am | 100–800 kW (60% of July) | Slow charge begins | Running 180 kW | ~500 kW | Off |
| 10am–4pm | ~1,200 kW (cloudy) | Reaches 70–80% only | Running 14hr today | ~700 kW | Import 10–20 MWh this month |
| 4pm–midnight | 500 kW declining then 0 | Partial discharge | Running extended hours | ~600 then 450 kW | Off most hours |
| KEY DIFFERENCE | Solar 54% of July output | BESS not fully charged | Genset runs 14hr vs 8hr normal | All loads still met | <3 MWh import total month |

KENYAN SEASONAL METEOROLOGY: Kenya experiences a bimodal rainfall pattern — the Long Rains (March–May, peaking in April) and the Short Rains (October–December) — which significantly reduces solar irradiance during these months. April is the design stress month: cloud cover can cut daily irradiance from 5.14 kWh/m²/day down to 2.5–3.5 kWh/m²/day for extended periods. Instead of economically ruining the project by oversizing the battery bank by 300% to survive these cloudy weeks, Angawatch realistically utilises a grid-interactive safety net (importing ~22 MWh/year from KPLC during the long rains) and extends the biogas genset operation from 8 hours/day to 14 hours/day during April. Biogas feedstock in April is actually HIGHER than normal because the rains increase organic material production from YattaFarm — the system is naturally more resilient exactly when it is most stressed. No load shedding occurs in any scenario modelled.

SECTION 09 SOLAR PV SYSTEM — COMPLETE TECHNICAL DESIGN

Solar PV is the primary generation source at Angawatch. It is not retrofitted onto existing houses — it is the reason the buildings are oriented and structured the way they are. The roof pitch, orientation, structural loading, cable conduit routing, and inverter room placement are all determined by the solar requirement from Day 1 of design.

## 9.1 Energy Alternatives Evaluated

| SOURCE | CONSIDERED | VERDICT | REASON |
| --- | --- | --- | --- |
| Solar PV (rooftop + carport + school) | Yes | SELECTED — 3,360 kWp | Irradiance 5.14 kWh/m²/day (Nairobi met station, Sunny Design validated). 7,680 × JKM425N-54HL4R-BDB 425W panels. No agricultural land used. |
| Community Biogas | Yes | SELECTED — Institutional cooking + 254 MWh/year electricity | 773 m³/day total yield (40-cow dairy upgrade). 539 m³/day to genset = 254 MWh/year. BCR ~2.8. Best BCR in project. |
| Yatta Canal Micro-Hydro | Yes | REMOVED — canal unsuitable for power | Canal flow 1.2–2.5 m³/s dry season, flat gradient. Maximum achievable power: 1–5 kW (Gemini research, 2026). Not 120 kW as previously claimed. Canal used for gravity irrigation ONLY — its original engineered purpose. |
| Concentrated Solar Thermal (CST) | Yes | REJECTED | CST requires Direct Normal Irradiance (DNI) > 5.5 kWh/m²/day. Yatta GHI 5.18 kWh/m²/day — DNI is lower due to atmospheric diffusion. PV is superior and simpler at this scale in 2026. |
| Thwake Dam PPA (20 MW hydro) | Considered | NOT SELECTED — excellent grid backup | Thwake Dam 50km away now operational (2026). Grid stability improved for whole Yatta region. Supports the KPLC connection quality but not a direct community power source. |
| Wind VAWTs | Yes | REMOVED | No reliable wind data at 1.17°S, 37.45°E. LCOE estimated KES 4,952–8,000/kWh vs solar KES 2.45. Solar superior in every metric. |
| Geothermal | Considered | NOT APPLICABLE | Geothermal resources limited to Rift Valley. Yatta Plateau is volcanic plateau rock, not rift. Not viable. |
| Green Hydrogen | Yes | REMOVED | EPRA H2 regulations in draft 2025. No Kenyan supply chain. Explosive storage risk at community scale. Replaced by biogas pipeline. |
| KPLC Grid (primary) | Considered | BACKUP + EXPORT ONLY | KES 25/kWh vs community solar KES 9/kWh. Grid retained for emergency backup and net metering export income only. |

## 9.2 Panel Specification — Jinko Solar JKM425N-54HL4R-BDB Tiger Neo N-Type (Sunny Design Validated)

SUNNY DESIGN VALIDATION: The original panel specified was the JKM415M-54HL4-B (415W, 2021 model). Sunny Design’s panel database includes the upgraded 2024 model: JKM425N-54HL4R-BDB Tiger Neo N-Type Bifacial 425W. This is a newer, better version of the same panel family — N-Type cell technology, higher efficiency, bifacial gain. Using this upgraded panel reduces count from 7,904 to 7,680 for the same 3,360 kWp installed capacity. Sunny Design independently calculated annual yield of 5,479.85 MWh — validating the design within 0.6% of hand calculations.

| PARAMETER | VALUE @ STC | VALUE AT YATTA OPERATING CONDITIONS | NOTES |
| --- | --- | --- | --- |
| Maximum Power (Pmax) | 415W | 358W at 70°C cell temp | Derating: -0.35%/°C × 45°C = -15.75% |
| Open Circuit Voltage (VOC) | 37.2V | 32.5V at 70°C | TempCoeff: -0.28%/°C |
| Short Circuit Current (ISC) | 11.14A | 11.39A at 70°C | TempCoeff: +0.05%/°C |
| Max Power Voltage (VMPP) | 30.9V | 26.03V at 70°C | Critical for MPPT range check |
| Efficiency | 21.3% | — | PERC monocrystalline — best available in Kenya via Solar World Kenya |
| Dimensions | 1722 × 1134 × 30mm | — | 1.953m² active area. Weight: 21.5 kg |
| Price (landed Nairobi bulk) | ~KES 8,500/panel | — | Including VAT, transport. Available Solar World Kenya, Mombasa Road. |
| Warranty | 25-year linear power | — | 0.55% annual degradation from Year 2 |

## 9.3 Series String Design — Temperature Safety Calculations

Before wiring panels in series, string voltage must be checked against inverter DC input limit at coldest operating temperature. Cold panels have HIGHER voltage than STC rated.

VOC at min temp = VOC_STC × (1 + |TempCoeff_VOC| × (T_min − T_STC))

VOC_max = 37.2 × (1 + 0.0028 × (12°C − 25°C)) = 37.2 × 1.0364 = 38.55V per panel

VMPP at max temp = VMPP_STC × (1 + TempCoeff_Pmax × (T_cell_max − T_STC))

VMPP_min = 30.9 × (1 + (-0.0035) × (70°C − 25°C)) = 30.9 × 0.8425 = 26.03V per panel

| CONSTRAINT | FORMULA | RESULT | VERDICT |
| --- | --- | --- | --- |
| Max panels in series (cold VOC limit) | 1,500V ÷ 38.55V/panel | MAX 38 panels/string | 1,500V inverter DC limit not exceeded |
| Min panels in series (hot VMPP limit) | 200V ÷ 26.03V/panel | MIN 8 panels/string | MPPT always tracking |
| CHOSEN STRING LENGTH | — | 38 panels per string | Maximum within safety limit → minimises cable cost |
| String VOC at min temp | 38 × 38.55V | 1,464.9V | < 1,500V ✓ |
| String VMPP at max temp | 38 × 26.03V | 989.1V | Above 200V minimum ✓ |

## 9.4 Parallel Configuration and Inverter Specification

| PARAMETER | VALUE | NOTES |
| --- | --- | --- |
| Inverter model (specified) | Huawei SUN2000-100KTL-M1 | 10 MPPT inputs, 100 kW AC rated, 1,500V DC max. Sunny Design validation used SMA SHP 100-20 400V (same power class, 98.8% efficiency vs Huawei 98.6%) — results are interchangeable. |
| MPPT inputs used per inverter | 4 of 10 | 6 reserved for future expansion |
| Strings per MPPT (parallel) | 2 | 2 × ISC = 22.78A < 26A MPPT limit ✓ |
| Panels per inverter | 8 strings × 38 panels = 304 |  |
| DC power per inverter | 304 × 415W = 126.2 kW |  |
| DC/AC ratio | 126.2 ÷ 100 = 1.26 | Acceptable range 1.1–1.5 ✓ |
| Inverters required (Sunny Design validated) | 40 × SMA SHP 100-20 400V (or Huawei SUN2000-100KTL-M1 equivalent) | 40 × 192 panels = 7,680 total panels. String: 24 panels × 8 strings per inverter. Max DC voltage 973V safely below 1,000V limit, verified in Sunny Design. Previous spec of 26 inverters was based on old 38-panel strings — corrected to 40 inverters per Sunny Design validation. |
| Effective installed capacity (with 3% bifacial gain) | 3,264 kWp base × 1.03 bifacial = 3,360 kWp effective | Sunny Design reports 3.36 MWp peak power — the difference from 3,264 kWp base reflects the bifacial gain factor of 3% applied in the simulation. |
| DC/AC ratio per inverter | 81.6 kWp ÷ 100 kW = 0.82 | Slightly under-ratio but Sunny Design nominal power ratio shows 121% — confirmed acceptable for this latitude and irradiance profile. |
| Inverter price (landed Kenya) | ~KES 480,000/unit | Total: KES 19.2M for 40 units (+KES 6.7M vs old 26-inverter spec, absorbed within KES 319M PV system CAPEX) |
| Annual generation | 5,480 MWh/year | Sunny Design independently validated at 5,479.85 MWh/year. PR 0.82 (vs conservative 0.77 hand calculation). Irradiance 5.14 kWh/m²/day from Nairobi station 71km away. |

## 9.5 Physical Panel Layout — Where the Panels Go

| LOCATION | USABLE AREA | CAPACITY | PANEL TYPE | NOTES |
| --- | --- | --- | --- | --- |
| Apartment flat rooftops (buildings) | 18,000m² | 1,980 kWp | Monofacial PERC | Flat at 10° tilt for self-cleaning. Row spacing prevents mutual shading. |
| EV carport canopies | 4,000m² | 680 kWp | Bifacial PERC | Elevated. Rear albedo from concrete below. Shades vehicles. |
| Primary school (tilted roof) | 1,000m² | 160 kWp | Monofacial | South-facing tilt |
| Secondary school | 1,000m² | 160 kWp | Monofacial | South-facing tilt |
| CHC rooftop | 400m² | 64 kWp | Monofacial |  |
| Market canopies + Jua Kali roofs | 800m² | 120 kWp | Monofacial |  |
| Boda swap station canopy | 200m² | 20 kWp | Bifacial | Charges swap packs directly |
| Other commercial / community roofs | 600m² | 96 kWp | Monofacial |  |
| TOTAL | ~26,000m² usable (base) / 3% bifacial gain on bifacial panels | 3,360 kWp effective (Sunny Design validated) | — | No agricultural land used for panels. 3,360 kWp = 3,264 kWp base + 3% bifacial gain on JKM425N bifacial panels. |

SECTION 10 COMMUNITY BIOGAS SYSTEM — COOKING PRIMARY, ELECTRICITY SECONDARY

CORRECTED FRAMING: Earlier versions of this document claimed biogas would supply cooking gas to all 2,000 households. This was based on a 4× overestimate of methane yield from human waste and sawdust. The honest calculation gives 773 m³/day of usable biogas (with a 40-cow commercial dairy lease) — enough to supply institutional cooking (CHC, schools, community market) and run the Cummins genset for 254 MWh/year of electricity. Individual homes use community SACCO bulk-procured LPG in Phase 1, transitioning to solar induction in Phase 3. The biogas system’s BCR of ~2.8 is still the best in the project, based on institutional cooking savings + electricity generation.

## 10.1 Corrected Feedstock Mass Balance — Honest Per-Source Yields

The previous version used a blanket yield of 0.45 m³/kg VS for all feedstocks. This was incorrect. Human waste has already had most of its energy extracted by the digestive system — its real methane yield is 0.25 m³/kg VS. Sawdust is lignocellulosic and resists anaerobic digestion at 0.20 m³/kg VS. The corrected calculation shows why a supplementary feedstock source (privately leased zero-grazing dairy) is necessary to reach viable gas volumes without relying on a cooperative-owned pig unit.

| SOURCE | INPUT kg/day | DM% | VS% | VS kg/day | REAL YIELD m³/kg VS | GAS m³/day | NOTES |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Human faecal waste (~8,000 residents × 200g) | 1,600 | 18% | 75% | 216 | 0.25 (low — human digestive system pre-extracts energy) | 54 | Via sewage pipeline direct to digester |
| Food prep waste (kitchens + schools + market) | 2,000 | 25% | 90% | 450 | 0.55 (good — food waste is highly digestible) | 248 | Vegetable peels, cooked waste, market leftovers |
| YattaFarm crop residues | 1,800 | 35% | 85% | 535 | 0.40 (moderate — lignocellulosic but digestible) | 214 | Stems, rejected produce. Seasonal ±30%. |
| Jua Kali sawdust/wood chips | 400 | 85% | 95% | 323 | 0.20 (low — wood resists anaerobic digestion) | 65 | Carpentry bays only |
| Commercial dairy lease (40 cows × 15 kg/day) | 600 | 80% | 80% | 384 | 0.40 (dairy manure — well-established in biogas production) | 154 | 40-head zero-grazing dairy is a bankable commercial operation, not a backyard hobby. 600 kg/day manure = 154 m³/day biogas — a meaningful baseload of methane. |
| TOTAL (40-cow commercial dairy) | 6,420 kg | — | — | 1,984 VS kg | Weighted avg 0.40 | ~735 m³/day | Significant increase from 5-cow version (600 m³/day). 40-cow commercial dairy adds 154 m³/day — a real, meaningful baseload. Total well above the 621 m³/day needed (80 cooking + 461 genset + 80 reserve). |

## 10.2 Commercial Dairy Lease — The Smart Way to Get Manure Without Doing the Work

CONTEXT: Machakos County is one of Kenya’s most active livestock farming areas. The Yatta Plateau is renowned for indigenous goat farming, and there has been a massive recent surge in zero-grazing dairy goats (Toggenburg and Alpine breeds) prized for high milk yield on minimal water and space. Dairy cows (Friesian and Ayrshire) also thrive at Yatta’s 1,450m elevation wherever reliable water exists. With the Yatta Canal and the community’s 15,000m³ retention pond, Angawatch has exactly the water security that makes a premium dairy operation attractive to an agri-entrepreneur.

THE MODEL: Angawatch does not own the animals. The cooperative takes 1 acre at the YattaFarm boundary and offers it to a private dairy farmer under a Commercial Lease Agreement. This is one of the most strategically elegant decisions in the project: the cooperative gets free daily manure delivered to the biogas plant; the farmer gets secure land, guaranteed water, and a location adjacent to a captive market of 8,000 potential dairy customers. Zero work for the cooperative. Strong local economic empowerment. No cultural or religious governance conflict.

| COMMERCIAL LEASE TERM | DETAIL |
| --- | --- |
| What Angawatch provides to the farmer | 1 hectare of secure fenced land at YattaFarm boundary for zero-grazing unit. Guaranteed daily water from the 15,000m³ retention pond. CRITICAL: 5 hectares of the 20-hectare YattaFarm is dedicated to growing canal-irrigated Napier grass (Pennisetum purpureum) and Brachiaria, sold to the dairy farmer at cost. At 50 tonnes/hectare/year yield, 5ha produces 685 kg/day dry matter — sufficient to feed 40 Friesians (daily need: 40 × 17 kg = 680 kg/day). This on-site fodder supply insulates the farmer against dry-season hay price spikes and guarantees the cooperative’s biogas feedstock supply. Return of ALL biogas digestate as organic fertiliser for the 20ha YattaFarm + 5ha Napier fodder plot. IMPORTANT: digestate stays on-site — it is NOT sold to external farms. The financial benefit (KES 14.5M/year in avoided chemical fertiliser) is classified as an OPEX saving. Connection to community market for dairy sales. |
| What the farmer provides to Angawatch | Small land lease fee (KES 5,000–8,000/month). GUARANTEED daily delivery of 100% of animal manure directly into the biogas digesters — written into the lease agreement, not optional. |
| Animal scale — commercial dairy | 40 Friesian or Ayrshire cows in a zero-grazing indoor unit on 1 leased hectare. Each produces 12–18 kg manure/day. Total: 600 kg/day. This is a proper commercial dairy operation — bankable, professionally managed, not a 5-cow backyard hobby. |
| Milk revenue — 40 cows | 40 cows × 4,000 litres/year = 160,000 litres/year. At KES 55–65/litre through community market + CHC cafeteria + external traders: KES 8.8–10.4M/year gross revenue to the private farmer. A 40-cow dairy is a serious Kenyan agri-business — exactly the kind of commercial tenant who will sign and honour a long-term manure supply agreement. |
| Biogas from 40-cow dairy (600 kg/day) | 600 × 0.80 VS × 0.40 m³/kg VS = 192 m³/day contribution to digesters — 7× more than 5 cows |
| Total biogas with 40-cow dairy | 581 (organic waste streams) + 192 (dairy) = 773 m³/day — 25% above minimum requirement of 621 m³/day. The dairy provides genuine feedstock reliability, not a rounding error. |
| FODDER SECURITY — why 5ha grass matters | 40 Friesians need ~680 kg dry matter/day. Buying hay externally in Machakos dry season (June–September) costs KES 8,000–15,000/tonne. If the farmer cannot afford fodder, cows leave, biogas collapses. The 5ha on-site Napier/Brachiaria plot — canal-irrigated from the 15,000m³ pond — produces exactly 685 kg/day dry matter, matching the herd’s full need. On-site fodder at cost (vs market rate) is the single strongest incentive in the lease deal. Farmer thrives. Biogas is secure. |
| Why 40 cows, not 5 | 5 cows produce 60 kg/day and 19 m³ of biogas — a rounding error for an 8,000-person settlement. A DFI financier or engineering judge would rightly call it a backyard hobby. 40 cows produce 600 kg/day and 154 m³/day biogas — a genuine commercial contribution that stabilises the biogas system. On 1 leased hectare with guaranteed water and a captive market of 8,000 dairy customers, a 40-cow dairy is a highly bankable business for a Kenyan agri-entrepreneur. |
| Cultural and religious fit | Dairy farming is universally respected across Muslim, SDA, Catholic, and ACK communities in Machakos County. No objections from any religious group. Dairy is a flagship Kenyan agricultural sector — judges will view this as appropriate and culturally intelligent. |
| Local economic empowerment | One lease agreement creates a serious agri-business opportunity for a Kenyan farmer. This is the kind of embedded local enterprise creation that competition judges specifically look for: not charity, not dependency, but a genuine business transaction that benefits both parties. |
| Total biogas vs requirement | 773 m³/day available vs 621 m³/day required (80 cooking + 461 genset + 80 reserve). Surplus: 152 m³/day. This surplus can either run the genset for additional hours or provide headroom for feedstock variability. The 40-cow dairy makes the system genuinely robust, not marginal. |

## 10.3 Gas Allocation — Honest Volumes

| ALLOCATION | VOLUME/DAY | PURPOSE | ANNUAL ENERGY |
| --- | --- | --- | --- |
| Institutional cooking (CHC kitchen + 2 school canteens + community market kitchen) | 80 m³/day | Replaces charcoal/LPG at 5 institutional kitchens. Near-zero PM2.5. | 480 MJ/day thermal |
| Electricity generation (balance to Cummins C220N5 genset) | 539 m³/day | Night baseload + extended hours possible. At same efficiency ratio as before (217 MWh from 461 m³/day): 539/461 × 217 = 254 MWh/year. | 254 MWh/year |
| Reserve + H2S scrubber losses + safety flaring (20%) | 154 m³/day (~20%) | Safety relief. At 773 m³/day production, 20% reserve = 154 m³/day. Higher absolute reserve with more gas = better system resilience. | — |
| TOTAL | 773 m³/day | — | — |

BIOGAS ELECTRICITY REVISED UPWARD: With 40-cow commercial dairy adding 154 m³/day, total biogas is 773 m³/day. After 80 m³/day institutional cooking and 154 m³/day reserve (20%), 539 m³/day reaches the genset. Annual electricity: 539/461 × 217 = 254 MWh/year. Revised totals: solar 5,480 (Sunny Design validated) + biogas 254 = 5,734 MWh/year. Demand 4,355 MWh. Export surplus: 1,379 MWh/year = KES 16.5M/year.

## 10.3 Digester and Equipment Specification

| COMPONENT | SPECIFICATION | RATIONALE |
| --- | --- | --- |
| Digester type | Fixed-dome Deenbandhu model | No moving parts. 30-year lifespan. Proven by SNV across Kenya. |
| Number of digesters | 6 × 100m³ fixed-dome | Total 600m³ active volume. 2 operate while 1 is maintained at any time. |
| Hydraulic retention time | 25 days | 600m³ ÷ 24m³/day average input |
| Operating temperature | 28–35°C (mesophilic) | Matches Yatta ambient — no external heating required |
| H2S scrubber | Iron sponge (Fe2O3 media) | Reduces H2S from ~1,500 ppm to <5 ppm. Mandatory for cooker safety and genset protection. |
| Gas storage | Flexigas 600m³ flexible bag | 5–15 mbar. Buffers day/night demand imbalance. |
| Power genset | Cummins C220N5 modified for biogas fuel | 220 kVA / 180 kW at 0.8 PF. ~500 m³ biogas/hr at full load. Operational in Kenya at JKUAT and Nakuru institutions. |
| Distribution pipe | HDPE SDR11 20mm ID | NOT copper — biogas attacks copper fittings. Low pressure 20–30 mbar. |
| Household gas meter | Elster BK-G4 diaphragm meter | Biogas-rated. Records consumption per unit for billing. |
| Gas sensor per kitchen | Honeywell BW Solo (CH4 + CO dual) | Alarm at 10% LEL, solenoid auto-shutoff at 20% LEL. |
| EPRA licence required | Gas Distribution Network Licence | Application 6 months before commissioning. Regulation exists to enable this type of project. |

## 10.4 Biochar from Digestate — Carbon-Negative Innovation

The solid fraction of biogas digestate (approximately 1.5–2 tonnes/day after gas extraction) is pyrolised on-site in a simple low-cost kiln at 400–600°C to produce biochar. ALL biochar is applied to the YattaFarm’s own soil — it is not sold externally. The agronomic logic: biochar permanently improves the sandy Yatta basaltic laterite soils by increasing water-retention capacity (reducing irrigation demand by 15–20%), sequestering carbon for 100+ years, and reducing the already-zero chemical fertiliser need further. Selling biochar externally was previously modelled as a KES 5.5M revenue line but has been correctly removed: a closed-loop eco-farm cannot mine its own soil’s carbon for a marginal cash payment while simultaneously claiming soil-health benefits. The on-site biochar application strengthens the YattaFarm’s KES 22M/year food crop and KES 3.4M chrysanthemum yields — the indirect benefit far exceeds the KES 5.5M it would raise if sold.

## 10.5 Biogas Safety — Is It Feasible at This Scale?

The biogas pipeline connects to: CHC (sterilisation, kitchen, staff room), Primary School (lunch kitchen), Secondary School (lunch kitchen + chemistry labs), Community market kitchen, and Cummins C220N5 genset for electricity. Individual homes are NOT on the biogas pipeline — this was corrected after honest feedstock analysis. SNV Kenya has proven 50-home community biogas in Western Kenya. 2,000 homes would require 10× more gas than honest waste streams can produce.

## 10.6 Phase 3 — Solar Induction Cooking for Homes (Year 10+)

From Year 10 onwards, as the community’s solar surplus grows and panel degradation is offset by BESS optimisation, individual homes transition from LPG to solar induction cooking. This eliminates the last fossil fuel dependency and reduces household Scope 1 emissions to zero.

| PARAMETER | VALUE | NOTES |
| --- | --- | --- |
| Induction hob model | Ramtons RM/500 2-plate induction cooker | Available nationwide in Kenya. KES 3,500–5,000 retail. 1,800W + 1,500W plates. |
| Daily household energy (cooking) | 1.1 kWh/day (family of 4, 3 meals) | Induction is ~90% efficient vs LPG ~55% — requires less energy for same cooking |
| Additional community load (2,000 homes) | 1.1 × 2,000 × 365 = 803 MWh/year | Added to Year 10+ demand |
| Year 10 solar surplus available | ~1,379 MWh/year (conservative — Sunny Design validated, panels slightly degraded by Year 10) | 803 MWh/year cooking < 1,379 MWh/year surplus. Absorption confirmed. |
| Monthly LPG saving per family | KES 1,295/month (LPG) → KES 450/month (solar induction electricity at KES 9/kWh × 1.1 kWh × 30) | Additional saving: KES 845/month from Year 10 |
| Carbon impact | LPG Scope 1: 0.288 tonnes/year → 0. Full elimination of household fossil fuels. | Household carbon drops to ~0.07 tonnes/year (residual KPLC import). 97.9% total reduction. |

SECTION 11 BATTERY ENERGY STORAGE SYSTEM — LFP CATL ENERONE

## 11.1 Chemistry — Why LFP Not NMC

| PROPERTY | LFP (SELECTED) | NMC (REJECTED) | WHY LFP WINS |
| --- | --- | --- | --- |
| Thermal runaway temperature | 270°C+ | 210°C | Critical: Yatta ambient 32°C peak. LFP safer margin. |
| Cycle life (to 80% capacity) | 4,000–6,000 cycles | 1,500–2,500 cycles | LFP lasts 2–3× longer at daily cycling |
| Safe depth of discharge | 90% | 80% | More usable energy from same rated capacity |
| Cost per kWh (2026) | ~USD 120–150/kWh | ~USD 145–175/kWh | LFP cheaper AND safer |
| Cobalt content | Zero | High | No conflict mineral supply risk |

## 11.1b Why Induction Cooking Is NOT in the Evening Battery Budget

A critical design clarification before reading the BESS sizing: the overnight battery allowance of 1.8 kWh per household is sized for BASE LOADS ONLY — lighting, electronics, refrigeration, fans, and television. It does not include cooking loads. This is deliberate and essential.

- The arithmetic of evening cooking: 2,000 households × 1,800W induction hob × 1 hour cooking = 3,600 kWh drawn in a single evening hour. At 7pm, with zero solar generation, this must come entirely from BESS. Providing this from battery storage would require 3,600 kWh additional BESS capacity — roughly 18 additional CATL EnerOne units at KES 100M more. The project cost would increase by KES 100M to solve a problem that SACCO LPG already solves at zero capital cost.

- The correct architecture: LPG handles thermal loads (cooking, water heating backup). Solar handles electrical loads (lighting, devices, refrigeration). BESS bridges the overnight electrical gap. This three-way separation — LPG for thermal, solar for electrical, BESS for overnight — is the defining cost-reduction insight of the Angawatch energy design. Any system that tries to use batteries for cooking is over-engineered and over-priced for a housing affordability project.

- Phase 3 solar induction cooking (Year 10+): When the community has 5+ years of carbon credit income and the BESS has been optimised, individual household induction cooking can be absorbed by the growing solar surplus during the 10am–2pm peak window. A family’s 1.2 kWh/day induction cooking load (scheduled by the M-Pesa SmartMeter IoT system to solar peak hours) replaces the LPG cylinder entirely. But this transition only makes economic sense when solar surplus is confirmed growing — not in Year 1 when every kWh of surplus has revenue value as a grid export.

BESS BASE LOAD DESIGN: The 1.8 kWh/household overnight allowance covers: LED lights (0.18 kWh), ceiling fan (0.20 kWh), refrigerator (0.43 kWh), TV (0.15 kWh), phone charging (0.04 kWh), WiFi router (0.24 kWh), and miscellaneous electronics (0.55 kWh). Total: 1.79 kWh ≈ 1.8 kWh. LPG covers: all cooking (2-burner hob, 1.2 kWh equivalent thermal per day). Solar thermal covers: all hot water. This three-way split is the honest design.

## 11.2 Overnight Demand and BESS Sizing

| NIGHTTIME LOAD (18:00–06:00) | AVG POWER (kW) | 12-HR ENERGY (kWh) | SOURCE |
| --- | --- | --- | --- |
| Residential (70% of daily load at night) | 270 | 3,240 | BESS primary |
| CHC critical loads (critical sub-circuit) | 18.8 | 226 | BESS priority |
| Cold storage (24/7) | 19.5 | 234 | BESS |
| Street lighting (overnight) | 10 | 120 | BESS |
| Water treatment (overnight) | 5 | 60 | BESS |
| EV overnight charging (5 buses) | 40.6 | 487 | BESS |
| Boda station (overnight restock) | 8.5 | 100 | BESS |
| TOTAL OVERNIGHT DEMAND | — | 4,467 | — |
| Less: biogas genset (180 kW × 8hr) | 180 | 1,440 | Biogas genset |
| NET BESS SUPPLY REQUIRED | — | 3,027 | BESS must supply |

BESS capacity needed (at 90% DoD) = 3,027 ÷ 0.90 = 3,363 kWh minimum

Add 20% safety margin: 3,363 × 1.20 = 4,035 kWh → use 18 × 200 kWh = 3,600 kWh

SIZING VERDICT: 18 × 200 kWh = 3,600 kWh installed provides 3,240 kWh usable at 90% DoD. This covers the 3,027 kWh net BESS requirement with 7% headroom. The full 20% safety margin target (which would require 4,035 kWh installed) is intentionally not achieved from batteries alone — it is met in practice by the biogas genset, which contributes 1,440 kWh nightly (180 kW × 8 hr) and can extend its run on high-demand nights. This three-way design — solar charges BESS, BESS covers base loads, genset fills peaks — avoids purchasing 12 additional CATL units (KES 67M) that would sit underutilised on 330+ days per year. The 3,600 kWh BESS is correctly sized for this architecture.

## 11.3 CATL EnerOne Specification — 18 Units

| PARAMETER | VALUE |
| --- | --- |
| Unit model | CATL EnerOne 200 kWh containerised |
| Cell chemistry | LFP (LiFePO4) — thermally stable, no runaway risk |
| Cells per unit in series | 192 cells × 3.2V nominal = 614.4V DC bus |
| Usable capacity per unit (at 90% DoD) | 180 kWh |
| Total system capacity (18 units) | 3,600 kWh installed / 3,240 kWh usable |
| Integrated bidirectional PCS | 100 kW per unit (AC/DC) |
| Total system power (18 × 100 kW) | 1,800 kW bidirectional |
| Cycle life | 6,000 cycles at 90% DoD = ~16.4 years at 1 cycle/day |
| Replacement year | Year 17 (within 20-year financial model) |
| All-in installed cost | USD 200–215/kWh = 3,600 × USD 207 × KES 130 = KES 96.9M → use KES 100M |
| Distribution | 18 units across 4 Energy Hubs (Hub A: 5, B: 5, C: 4, D: 4). One unit dedicated CHC. |

SECTION 12 HYBRID GRID STRATEGY — CITIZEN-CONTROLLED ENERGY

## 12.1 Energy Priority Hierarchy

Energy dispatch at Angawatch follows a strict citizen-benefit priority order: (1) Fill BESS from solar surplus during peak hours. (2) Power all community loads including industrial. (3) Charge EV fleet. (4) Export surplus to KPLC grid. No energy is exported until community needs are fully met. Residents control their own energy access — not a central government utility or algorithm.

## 12.2 Annual Grid Balance

|  | ANNUAL VOLUME | FINANCIAL IMPACT |
| --- | --- | --- |
| Total generation | 5,734 MWh/year | Solar 5,480 (Sunny Design validated 5,479.85 MWh) + Biogas 254 |
| Community demand | 4,355 MWh/year | Irrigation pump revised to 10kW/36.5 MWh for sloped site. Borehole pumps (60.2 MWh) explicit. |
| KPLC grid import (April + rare events) | ~22 MWh/year | 22,000 kWh × KES 25/kWh = KES 550,000/year |
| Grid export (net metering surplus) | ~1,379 MWh/year | 5,734 − 4,355 = 1,379 MWh × KES 12/kWh = KES 16.5M/year income. |
| NET POSITION | Community earns 30× more than it pays | Net income: +KES 16.0M/year from grid relationship (KES 16.5M export − KES 550K import) |

## 12.3 M-Pesa SmartMeter — Kenyan Energy Innovation

The Kaifa MA105 prepayment meter (same hardware used by KPLC for residential prepay) is installed in all 2,000 apartments. The community billing system replicates KPLC’s M-Pawa token architecture using Safaricom’s Daraja API — but at KES 9/kWh (community rate) instead of KES 25/kWh (KPLC rate).

| STEP | ACTION | TECHNOLOGY |
| --- | --- | --- |
| 1. Resident buys units | Dials *384# USSD shortcode or WhatsApp bot | Safaricom USSD / WhatsApp API |
| 2. Payment | Confirms via M-Pesa PIN | Daraja STK Push API |
| 3. Token generation | Community billing server: kWh at KES 9/kWh → 20-digit STS token | Open-source STS token engine (IEC 62056 compliant) |
| 4. SMS delivery | Token to resident phone in <15 seconds | Safaricom SMS API |
| 5. Token entry | Resident enters on Kaifa MA105 keypad | Standard STS protocol — same process as KPLC |
| 6. Net metering credit | Monthly kWh credit posted to resident M-Pesa wallet | Daraja B2C API |
| 7. Revenue collection | All payments to community SACCO Paybill account | MPESA Paybill API |

Software development cost: KES 2.5M. At 150 kWh/month per household (planned Angawatch consumption), each family pays KES 1,350/month vs KES 3,750 at KPLC for identical consumption — a saving of KES 2,400/month per family. Community-wide saving: KES 2,400 × 2,000 = KES 4.8M/month = KES 57.6M/year. Return on KES 2.5M software investment: 2,304% per year — still extraordinary. For comparison, a Nairobi satellite household consuming 460 kWh/month pays KES 11,500 at KPLC; at the community rate the same consumption costs KES 4,140 — a saving of KES 7,360/month. Angawatch households are designed to need only 150 kWh/month, achieving this comfort level at 64% lower cost than any KPLC-connected home.

SECTION 13 WATER CONSERVATION, SEWAGE TREATMENT &** STORMWATER**

Water is the second most critical resource after energy. Angawatch’s integrated water strategy covers supply (canal gravity, rainwater, borehole backup), treatment, greywater recycling, sewage treatment, and stormwater retention — a closed-loop system targeting 60% reduction in treated water demand versus an unplanned settlement.

## 13.1 Rainwater Harvesting — Corrected to 60,000m² Catchment

PREVIOUS VERSION ERROR: The catchment area was stated as 26,000m² — the same figure used for solar panel area. These are different surfaces. The correct impermeable roof catchment for rainwater harvesting: 1,280 apartment rooftops (~19,200m²) + 720 maisonette rooftops (~46,800m²) + schools, CHC, market (~6,000m²) + solar EV carport canopies (~5,000m²) = 77,000m² actual. Using 60,000m² as a conservative working figure: 60,000 × 0.65m × 0.85 = 33,150m³/year — 155% more than previously calculated.

| PARAMETER | VALUE | NOTES |
| --- | --- | --- |
| Total catchment area (revised) | 60,000m² (conservative) | Actual ~77,000m². Using 60,000m² to account for unusable/shaded areas. |
| Annual rainfall | 650mm/year (0.65m) | Average of bimodal 500–700mm range. Conservative. |
| Runoff coefficient | 0.85 | Standard for corrugated iron/tiled roofs. |
| Annual harvestable volume | 33,150m³/year | 60,000 × 0.65 × 0.85 = 33,150m³ (was 13,000m³ with wrong base) |
| Daily average | 90,800 L/day | 33,150 × 1,000 ÷ 365 |
| During 120 rainy days | 276,250 L/day | Concentrated in two rain seasons |
| Combined with boreholes (rainy season) | 676,250 L/day = 84.5 LPCD | Comfortable margin above WHO 50 LPCD |
| Overflow routing — CRITICAL | Community retention pond (7,500m³, Section 13.5) | NOT the YattaFarm agricultural pond — that is upslope. Roof overflow gravity-feeds to the community retention pond at ground level, then to landscaping and orchard. |
| Stage 1: First-flush diverter | First 2mm diverted to drain | Removes dust, bird droppings, debris before clean water enters system |
| Stage 2: Potable header tanks | 200,000L closed UV-treated tanks | Clean rain → potable supply. Fills rapidly in storm. |
| Stage 3: Cluster cistern overflow | 20 × 50,000L underground cisterns = 1,000m³ | When header tanks full: overflow to cluster cisterns for toilet flushing and garden use. |
| Stage 4: Community retention pond | 7,500m³ (at ground level, below buildings) | When cisterns full: overflow to community pond. A 50mm storm event generates 2,550m³; this cascaded system captures all of it with zero street flooding. |

## 13.2 Greywater Recycling

| PARAMETER | VALUE |
| --- | --- |
| Greywater generated per household/day | ~80 litres (4 persons × 20L shower/sink) |
| Community total | 160,000 litres/day |
| Reed bed recovery efficiency | 70% |
| Recycled for toilet flushing + garden | 112,000 litres/day |
| Replaces municipal water for toilets | Fully — 4 flushes × 6L × 4 persons = 96L/day per household |

## 13.3 YattaFarm Irrigation — Solar-Pumped from 15,000m³ Gravity-Fed Water Pan

IRRIGATION SYSTEM DESIGN: The Yatta Plateau slopes uphill from the canal — the site sits above the canal, not below it. This means water cannot flow from the canal to the farm by gravity. Instead, a 10kW solar high-lift pump at the canal boundary pumps water uphill during the wet seasons to fill the 15,000m³ retention pond at the farm’s highest elevation. Once full, the pond acts as a massive gravity header-tank: all drip irrigation lines run downhill from the pond to lower farm zones, driven entirely by gravity. The engineering logic: pump fills upward once per season, gravity distributes continuously for free. This is the correct physical model for an upslope site. The ‘gravity fill’ narrative used in earlier versions of this document was physically impossible given the site topography and has been corrected.

CANAL CAPACITY CONFIRMED: The Yatta Canal post-rehabilitation carries 3 m³/second = 259,200 m³/day. It is officially designed to irrigate 18,000 hectares of arable land in Yatta constituency. YattaFarm’s 20 hectares requires 1,000 m³/day = 0.4% of canal flow. Water availability is not a constraint.

The 15,000 m³ Water Pan — Dry-Season Security

A 15,000 m³ geomembrane-lined earthen water pan (100m × 50m × 3m deep, occupying 0.5 ha of YattaFarm) is the cornerstone of irrigation security. At 1,000 m³/day peak demand, 15,000 m³ provides a 15-day complete drought buffer. The pond occupies 2.5% of the 20-hectare farm — a minimal sacrifice for guaranteed water security.

| IRRIGATION SYSTEM COMPONENT | SPECIFICATION | NOTES |
| --- | --- | --- |
| Water pan volume | 15,000 m³ (100m × 50m × 3m deep) | 0.5 ha footprint. Verified: medium-sized reservoir, standard in East African commercial farming. |
| HDPE geomembrane liner | 0.5mm HDPE, 5,900 m² (base + sides) | Essential: Yatta basaltic laterite is moderately permeable. Unlined pond loses 10–30% to seepage monthly. KES 4.1M. |
| Canal intake pump at canal boundary | 10kW solar submersible pump at canal edge. 8-inch HDPE rising main to pond. | The Yatta Plateau slopes uphill from the canal. The pond sits at the HIGHEST elevation of YattaFarm, not below the canal. Water cannot flow uphill. The pump fills the pond. The full pond then gravity-distributes downhill to all drip irrigation zones. This is physically correct — pump fills, gravity distributes. |
| Pump fill rate to pond | 10kW pump at ~30m total head delivers ~50 litres/second = 180 m³/hour | Empty-to-full in 8.5 days of solar pumping. Solar pump runs 10 hours/day (8am–6pm) = 1,800 m³/day. 15,000 ÷ 1,800 = 8.33 days ≈ 8.5 days. Previous ‘3.5 days’ error assumed 24-hour pumping — incorrect for a solar-powered pump. |
| Seasonal pump operation | Pump runs during April–May and November–December wet seasons to fill pond | Canal at max flow (259,200 m³/day). Annual pumped volume: 30,000 m³ = trivial draw on canal. WRA-friendly: pump draws only when canal has maximum surplus. During dry season, pump is off — farm runs entirely from stored pond water by gravity. |
| Dry-season operation | Sluice gate closed. Farm runs entirely from stored pan water. | Leaves canal water for downstream farmers during low-flow months. Resolves the documented upstream-downstream conflict. |
| Stormwater harvest (bonus) | Contour swales direct runoff into pan during heavy rain | A 50mm storm across 10ha: ~1,500–2,000 m³ additional inflow at 0.4 agricultural runoff coefficient. Tops up pan for free. |
| 10kW high-lift pump at canal boundary | Grundfos SP 30-14 or equivalent, 10kW, lifts water ~30m from canal level to pond at farm’s highest elevation | Canal boundary location. 8-inch HDPE rising main. Fills pond during wet seasons. During dry season: switched off. Pond gravity-distributes to drip lines — no separate distribution pump needed. |
| Pond as gravity header | The 15,000m³ pond at the highest farm elevation replaces the need for a separate elevated header tank. The pond’s hydraulic head drives gravity distribution to all drip lines on the downslope farm. One large header is more robust than a 200,000L steel tank. |  |
| Sand-gravel filter | At header tank inlet, before chrysanthemum drip lines | Canal silt and algae block drip emitters. Filter essential for 4ha chrysanthemum plot. KES 180,000. |
| Annual pumping energy | 10 kW × 10hr/day × 365 = 36,500 kWh = 36.5 MWh/year | Revised up: 10kW pump required for ~30m total dynamic head on sloped Yatta Plateau site. Previous 5kW spec assumed flat 15m head — insufficient for actual topography. |
| Pump energy source | 100% solar during daytime — no BESS draw | Scheduled 8am–6pm. 10kW falls within peak solar window. No additional BESS sizing required. Solar surplus (1,379 MWh/year — Sunny Design validated) comfortably absorbs the additional 36.5 MWh/year. |

WRA permit covers: (1) Canal off-take sluice for seasonal pond filling (30,000 m³/year). (2) Groundwater abstraction permit for boreholes (potable supply — separate permit). Annual abstraction of 30,000 m³ from a canal carrying 94 million m³/year is environmentally trivial and will receive straightforward WRA approval.

## 13.4 Sewage Treatment Chain — Source Separation Model

CRITICAL ENGINEERING CLARIFICATION: Blackwater (toilet waste) and greywater (sinks/showers) are kept COMPLETELY SEPARATE from the point of generation. Blackwater goes directly to the centralised biogas plant — it never returns to the residential cluster area. Only greywater is handled within the Ekeja cluster courtyard reed beds, because greywater is low-strength, low-pathogen wastewater that is safe and appropriate for proximity to residential areas.

STREAM 1 — Blackwater Treatment (toilet waste) — Centralised, Remote

- Step B1 — Source separation at WC: Blackwater collected in dedicated HDPE gravity sewers (separate from greywater drain). No shared pipework between streams.

- Step B2 — Biogas digesters (primary treatment): Raw blackwater → 6 × 100m³ Deenbandhu fixed-dome digesters. 25-day retention. Anaerobic digestion destroys 90%+ of pathogens. Produces biogas (fuel + electricity) + digestate.

- Step B3 — Digestate liquid effluent: Liquid fraction (~1,080m³/day after digestion) is piped by gravity to the REMOTE constructed wetland at the YattaFarm boundary — minimum 200m from the nearest residential block. Never returns to cluster courtyard area.

- Step B4 — Remote constructed wetland (YattaFarm boundary): Large-scale Phragmites australis (common reed) beds. 48-hour hydraulic retention. 85% BOD reduction. Located downwind and downslope from housing. Maintenance access by farm staff, not residents.

- Step B5 — Slow sand filter + irrigation pond: Wetland effluent → 500mm sand filter → 5,000m³ irrigation pond → YattaFarm drip irrigation for non-food-contact crops only (chrysanthemums, fuel crops). NEVER for vegetable or food crop irrigation.

STREAM 2 — Greywater Treatment (sinks/showers) — Within Ekeja Courtyard

- Step G1 — Courtyard reed bed per Ekeja cluster: Sink and shower greywater from each cluster (low-strength, low-pathogen) feeds a small constructed reed bed in the courtyard garden area. 24-hour retention. 70% BOD reduction.

- Step G2 — Greywater recycled for toilet flushing: Reed bed outflow → communal storage cistern → toilet cisterns in the cluster. Replaces potable water for WC flushing entirely. Net saving: 96 litres/day per household.

- Step G3 — WHY GREYWATER IN THE COURTYARD IS SAFE: Greywater has no faecal contamination, low pathogen load, and reed bed treatment makes it safe for proximity to residential areas. This is standard practice in East African eco-developments. Blackwater is NEVER routed through residential courtyard systems.

STREAM 3 — Potable Water: 50 LPCD Boreholes + Rainwater Top-Up

BASELINE: 3 × deep boreholes supply 400m³/day = 400,000 litres ÷ 8,090 residents = 49.4 litres per capita per day (effectively 50 LPCD) (LPCD). WHO minimum for basic services: 50 LPCD. For a middle-class community with full plumbing, this is tight — two teenagers showering for 10 minutes each would consume the household’s daily share. The solution: centralised rainwater harvesting on top of borehole supply.

| POTABLE WATER SOURCE | DAILY VOLUME | NOTES |
| --- | --- | --- |
| 3 × deep boreholes (Grundfos SP17-7, 5.5kW) | 400,000 L/day | 50 LPCD. Primary source year-round. Defluoridated through bone char filters. |
| Centralised rooftop rainwater harvesting (revised) | Up to 276,250 L/day during rainy seasons | 60,000m² catchment × 0.65m rainfall × 0.85 runoff ÷ 120 rainy days × 1,000 = 276,250 L/day peak. Previous figure (124,800 L/day) used incorrect 26,000m² base. Corrected by first-flush diverter → potable header tank → cluster cistern overflow → community retention pond cascade. |
| TOTAL (dry season — boreholes only) | 400,000 L/day | 50 LPCD — adequate with greywater toilet substitution (saves effective 24 LPCD) |
| TOTAL (rainy season — both sources) | 676,250 L/day | 84.5 LPCD — well above WHO 50 LPCD minimum. Borehole recharge time during dry months. |
| Effective LPCD with greywater substitution | Dry season: 50+24=74 LPCD equivalent. Rainy: 84.5 LPCD fresh + 24 grey = effectively 108 LPCD | Greywater (courtyard reed beds) covers all toilet flushing year-round. Potable water used for drinking/cooking/bathing only. Annual average: 61.4 LPCD from boreholes + rain — comfortably above WHO 50 LPCD. |

- Step P1 — Sources: (a) 3 × deep boreholes (Grundfos SP17-7, 5.5 kW each), pumped 8am–6pm solar. (b) Centralised rainwater: all apartment roofs + carport canopies guttered to centralised UV-filtered storage.

- Step P2 — Defluoridation: Bone char filters at centralised water treatment plant before distribution (volcanic fluoride endemic in Yatta basaltic aquifer). Bone char produced from YattaFarm animal bones — closes another material loop.

- Step P3 — Distribution: Elevated 200,000L storage tank gravity-feeds community potable supply. UV disinfection at tank outlet.

SUMMARY: Blackwater → remote biogas → remote wetland → farm. Greywater → courtyard reed bed → toilet flushing. No digestate or raw sewage ever enters the residential cluster area. This is the gold standard of source separation sanitation.

## 13.5 Stormwater Management

On 90 hectares receiving 700–900mm/year concentrated in two seasons, stormwater management is a real engineering requirement. A 50mm rainfall event on 90ha generates: 90 × 10,000m² × 0.05m × 0.70 runoff = 31,500m³ in a single storm event.

- Permeable paving on all pedestrian paths and Ekeja courtyards: water infiltrates, recharges groundwater, reduces surface runoff volume by 40%

- Bioswale drainage channels between building clusters: grass-lined channels slow flow, filter suspended solids, direct water to retention pond

- Community retention pond (0.5ha, 1.5m deep = 7,500m³): stores peak storm runoff. Overflow to YattaFarm irrigation channels.

- Retention pond doubles as irrigation buffer for YattaFarm during dry periods: gravity-fed drip irrigation from elevated pond position

- Green roof buffer on CHC and school flat roofs: 75mm substrate with indigenous succulent groundcover absorbs first 15mm of rainfall

## 13.6 Water Demand Reduction Summary

| STRATEGY | DAILY SAVING | ANNUAL SAVING (m³) |
| --- | --- | --- |
| Greywater recycling | 112,000 litres | 40,880 |
| Centralised rainwater harvesting (revised) | 90,800 litres avg / 276,250 litres peak | 33,150 m³/year (corrected from 13,000m³: catchment updated from 26,000m² to 60,000m²) |
| Drip vs flood irrigation (YattaFarm) | 30,000 litres | 10,950 |
| Stormwater capture and reuse | ~25,000 litres (avg day) | 9,125 |
| Solar water heating (no boiler purge waste) | 8,000 litres | 2,920 |
| TOTAL | ~210,616 litres/day | ~76,875m³/year |

SECTION 14 COMMUNITY INFRASTRUCTURE &** AMENITIES**

Angawatch is a complete community — not just housing. Every amenity reduces external resident spend or generates cooperative revenue. The community is designed to be a desirable place to live, not just an affordable one.

## 14.1 Community Health Centre — 50-Bed Level 3 Facility (1,500m²)

| AREA | m² | KEY FEATURES |
| --- | --- | --- |
| OPD + triage + waiting | 200 | 6 consultation rooms, 80-seat waiting, children’s area |
| Emergency / casualty | 120 | 2 resuscitation bays, 4 observation beds, ambulance access |
| General ward | 200 | 20 adult beds. Natural light east+west. Cross-ventilation. |
| Maternity + neonatal | 150 | 8 antenatal beds, 2 delivery rooms, 4 neonatal incubators (CRITICAL power) |
| Theatre + pre/post-op | 80 | 1 operating table, laminar flow, autoclave room |
| Laboratory | 80 | Haematology, biochemistry, malaria RDT, cold chain 2–8°C (CRITICAL power) |
| Radiology (digital DR X-ray) | 40 | Lead-lined. Digital reporting workstation. |
| Pharmacy + controlled drug safe | 60 | Dispensing counter. Critical cold chain. |
| Administration + EMR server | 100 | Records room, 4 workstations, CEO office |
| Staff changing + rest + ablutions | 100 | MoH guidelines compliance |
| BESS sub-circuit room + UPS | 60 | 100 kWh CATL unit dedicated to CHC. ATS. IT rack. |
| Corridors + toilets + store | 310 | Min 2.4m corridor width for trolley access |
| TOTAL | 1,500 |  |

## 14.2 Schools

| SCHOOL | PUPILS | TEACHERS | ENERGY | CAPITAL |
| --- | --- | --- | --- | --- |
| Primary School (20 classrooms) | 600 (Std 1–8, 30/class) | 24 (resident priority) | 100% solar + 300kWh BESS | KES 130M |
| Secondary School (30 classrooms + labs) | 800 (Form 1–4, ~26/class) | 40 (subject specialists) | 100% solar + labs | KES 195M |
| COMBINED | 1,400 students | 64 teachers | 37.6 MWh/year | KES 325M |

Secondary school includes Solar Energy practical module — community systems used as live study subject for Form 3 Physics and Chemistry students.

## 14.3 Place of Worship

Machakos County is 85%+ Christian (predominantly Catholic, ACK Anglican, and various Pentecostal). Not providing a worship space in a community of this size would be culturally inappropriate and practically unrealistic — residents would form informal worship groups regardless. Angawatch provides:

- Interdenominational Worship Centre: 800-seat multi-faith hall on 0.4ha within the Community Hub zone. Open to all denominations by rotational booking schedule. Construction: CSEB, corrugated iron roof with solar. Capital cost: KES 45M.

- Outdoor gathering space adjacent to worship centre: 2,000-person capacity for large congregation events, community meetings, and cultural ceremonies.

- Mosque provision: Prayer room (80-person capacity) within the worship centre for Muslim residents — Machakos has a 5–10% Muslim population.

## 14.3b Community Governance — How Angawatch Is Managed

Angawatch is owned and managed by the Angawatch Community Cooperative Society, registered under Kenya’s Cooperative Societies Act (Cap 490). Every resident family is a member. The cooperative holds title to all common property: energy infrastructure, YattaFarm, the market, CHC, schools, and community roads.

| GOVERNANCE ELEMENT | DETAIL |
| --- | --- |
| Legal structure | Registered cooperative society under Cap 490. Annual general meeting. 7-member elected board. |
| Membership | Automatic for all 2,000 unit holders (owners or renters of cooperative units). One vote per household. |
| Revenue distribution | Net cooperative revenue distributed: 40% to residents as annual dividend, 30% to infrastructure maintenance fund, 20% to cooperative expansion, 10% to Machakos County (public infrastructure contribution) |
| County government role | Minority stakeholder (10% equity). Provides land development approval, CHC staffing support, school teacher placement via TSC. |
| Energy governance | M-Pesa SmartMeter billing managed by cooperative. Community sets tariff by vote (currently KES 9/kWh). Residents are prosumers — they generate, consume, and earn from net metering. |
| First occupancy to 2030 | Project approval target: 2026–2027. Phase 3 first occupancy: Month 18 from approval = mid-2028. Full community: Month 42 = 2030. This meets the competition requirement of ‘construction completion and move-in of first units within four years of project approval, notionally before year end of 2030.’ |

## 14.4 Recreation and Sports

Children will live here from birth through secondary school (the competition brief specifies this). A community with no recreation infrastructure will fail to retain families. Angawatch provides:

- Full-size football pitch: 100m × 68m natural grass within green space zone. LED floodlighting (4 × 1,000W mast lights) for evening use — powered from community solar.

- Children’s playground per Ekeja cluster: 20 clusters × one playground = 20 playgrounds. Age 2–12 equipment: swings, climbing frames, slides. CSEB sand enclosure borders.

- Basketball/netball courts: 2 hard courts shared between secondary school and community. Dual use avoids duplication.

- Community running track: 400m gravel path around the green space perimeter. Zero energy cost. Community fitness programme.

## 14.5 Broadband and Digital Infrastructure

| COMPONENT | SPECIFICATION | NOTES |
| --- | --- | --- |
| Fibre to building (FTTB) | Safaricom or Jamii Telecom (Faiba) fibre | Both operate in Machakos County. 1 Gbps backbone to each Energy Hub. |
| Horizontal distribution | Cat6 ethernet or WiFi access point per floor | One access point per floor landing serves 4 apartments |
| Resident connectivity | 100 Mbps symmetric per apartment | Sufficient for telemedicine, KNEC exam marking, video calls |
| CHC connectivity | Dedicated 1 Gbps link to county health system | Telemedicine to Machakos Level 5 + KNH via Kenya Health Network |
| Smart grid monitoring | M2M IoT links on all BESS + inverters | Energy Management System (EMS) monitors all hubs centrally |
| Energy cost | Negligible — 10W router per apartment already in energy audit |  |

## 14.6 Solid Waste Management

2,000 families generate approximately 3–4 kg of solid waste per household per day = 6–8 tonnes/day community-wide. Composition:

| WASTE STREAM | % COMPOSITION | DESTINATION | ENERGY LINK |
| --- | --- | --- | --- |
| Organic (food + garden) | 55% | Biogas digesters — primary feedstock | Becomes institutional cooking gas + 254 MWh/year electricity + biochar. Homes on SACCO LPG. |
| Recyclable (plastic, glass, metal, paper) | 25% | Jua Kali zone material recovery — sorted and sold | Metal → fabrication. Plastic → block paving. Paper → composting. |
| Electronic waste | 2% | Designated e-waste collection point (WEEE Kenya) | Mohamed’s phone repair recovers components |
| Residual (non-recyclable) | 18% | Weekly county lorry collection to Machakos landfill | Minimised by active sorting culture |

- Waste sorting stations: one per Ekeja cluster. Covered structure. 4 bins: organic (green), recyclable (blue), e-waste (orange), residual (black). CSEB construction.

- No waste-to-energy incineration: Machakos County air quality is clean baseline — maintaining it is a design objective. Incineration would introduce PM2.5 from the one direction biogas eliminates it from.

## 14.7 YattaFarm — Canal-Irrigated Food Cooperative

| FEATURE | DETAIL |
| --- | --- |
| Total area | 20 hectares: 10.5ha food crops + 4ha chrysanthemums + 5ha Napier/Brachiaria dairy fodder + 0.5ha water pan = 20ha total. The 5ha fodder plot is canal-irrigated and supplies the 40-cow dairy lease at cost — securing biogas feedstock reliability. |
| Irrigation system | 15,000m³ geomembrane-lined water pan at HIGHEST elevation of YattaFarm. 10kW solar high-lift pump at canal boundary pumps water uphill into pond during wet seasons (April–May, November–December). Once full, the pond acts as a gravity header-tank: water flows downhill by gravity to ALL drip irrigation zones on the farm. No additional pump needed for distribution. Dry season: pump off, farm runs entirely from stored pond water by gravity. |
| Water pan infrastructure | HDPE geomembrane liner (KES 4.1M), 10kW high-lift pump + solar controller (KES 1.8M), 8-inch HDPE rising main from canal to pond (KES 0.6M), sand-gravel filter for chrysanthemum drip (KES 0.18M). Total: KES 6.68M. NOTE: No separate elevated header tank needed — the 15,000m³ pond at the farm’s highest elevation IS the header tank. KES 1.8M saved vs previous tank-and-tower design. |
| Annual pumping energy | 36.5 MWh/year (10 kW × 10hr/day × 365). 100% solar — scheduled 8am–6pm. 10kW required for ~30m total dynamic head on sloped site. |
| Food crops (16ha) | Sukuma wiki, tomatoes, onions, beans, sweet potato, green grams, herbs |
| Chrysanthemum plot (4ha) | Domestic cut flower market (Nairobi + Machakos). KES 850,000/ha/year. Revenue: KES 3.4M/year. Sand-gravel filtered supply — emitter protection. |
| Cold storage | 2 × 20-tonne cold rooms (Carrier 38AK). 171 MWh/year. Post-harvest loss < 5%. |
| Fertiliser | 100% from biogas digestate + biochar from pyrolysis. Zero chemical fertiliser purchase. |
| Labour | 40+ full-time cooperative members. Resident priority. Professional farm manager (agribusiness graduate). |
| Capital cost | KES 182M (farm + cold storage + irrigation infrastructure including water pan) |
| Revenue | KES 22M food crops + KES 3.4M chrysanthemums + KES 5.5M biochar = KES 30.9M/year |

## 14.8 E-Mobility Infrastructure — Roam Motors Fleet

| VEHICLE | SPEC | COUNT | DAILY USE | ENERGY |
| --- | --- | --- | --- | --- |
| Roam Rapid e-matatu | 68 kWh battery, 160km range, 0.35 kWh/km, KES 6.5M | 5 | 5:15am–9:00pm. Matuu–Machakos + internal. | 127 kWh/bus/day |
| Roam Air e-boda | 1.85 kWh swappable pack, 100–130km range, KES 175,000 | 50 | Swap station: 2-min swap. 1 swap/day average. | 2.04 kWh/boda/day |
| Battery swap station | 30-slot Ampersand-model, 20 kWp canopy solar, KES 2.8M | 1 | Charges 30 packs during day via direct solar | 30 kW average load |

EV operational schedule: First bus departure 5:15am (night BESS charge complete). Mid-day opportunity charge 8:15–9:00am (DC 50kW fast chargers from peak solar). Overnight BESS charging 9:00pm–6:30am. April stress month: mid-day charging window shorter (cloudy) — buses may do 3 return trips instead of 4 on worst April days.

## 14.9 Jua Kali Industrial Zone

| FEATURE | DETAIL |
| --- | --- |
| Workshop bays | 50 bays × 25m². Metal fabrication, carpentry, auto mechanics, tailoring, electronics repair. |
| CSEB block factory | On-site pressing from Yatta laterite. Supplies construction + external sale KES 4.8M/year. |
| Commercial strip | Ground-floor units all 30 blocks: shops, pharmacies, M-Pesa agents, restaurants. |
| Open market | Weekly market open to Yatta sub-county. 500+ stalls. |
| Harmonic protection | Active harmonic filters on Jua Kali sub-panel — arc welding generates severe THD (total harmonic distortion). Filters prevent harmonics propagating to residential grid. |

SECTION 15 REGULATORY FRAMEWORK — APPLICABLE LAWS AND CODES

Angawatch must comply with the following Kenyan and international regulatory instruments. All compliance milestones are incorporated into the Phase 1 pre-construction activities in Section 19.

## 15.1 Housing and Construction

| REGULATION | REQUIREMENT | ANGAWATCH COMPLIANCE |
| --- | --- | --- |
| Kenya National Building Code 2023 | Minimum room sizes, structural requirements, fire separation, ventilation, accessibility (KS 1515) | 3BR: all rooms exceed minimum sizes. Ground floor units wheelchair accessible. CSEB walls comply with KS 02-1070. |
| Physical Planning Act (Cap 286) | Development Permission from Machakos County Planning Dept before construction | Phase 1 activity: Development Permission application filed 3 months before groundbreaking. |
| Machakos County Land Regulations | Zoning compliance: residential + commercial + light industrial | Site zoning allows mixed-use development under Machakos County Spatial Plan 2023. |

## 15.2 Energy

| REGULATION | REQUIREMENT | ANGAWATCH COMPLIANCE |
| --- | --- | --- |
| Electricity Act Cap 314 | Grid connection, distributed generation licence, net metering registration | KPLC grid connection agreement. EPRA DG licence for 3,360 kWp (Sunny Design validated effective capacity including 3% bifacial gain). Net Metering Agreement under EPRA 2022 Regulations. |
| EPRA Net Metering Regulations 2022 | Export below 1 MW net capacity threshold. EPRA bilateral export agreement. | Net export ~1,308 MWh/year at ~150 kW average — within threshold. EPRA agreement required. |
| EPRA Gas Distribution Network Licence | Required for institutional biogas pipeline (CHC, schools, market) | Scope: 5 buildings, not 2,000 households. Significantly simpler approval than a residential gas network. EPRA precedent: existing small town gas networks in Eldoret. |
| Kenya Revenue Authority — VAT | Finance Act 2023 Schedule 2: solar panels, batteries, CSEB presses VAT-exempt. All other construction materials: 16% VAT. | Construction cost budgeted at KES 46,500/m² blended (VAT-inclusive). VAT on non-exempt materials: ~8% of total construction. KRA VAT registration required for cooperative as commercial entity. |
| Kenya Revenue Authority — PAYE | Cooperative is an employer. All salaried CHC staff, school staff, farm manager, cooperative employees: PAYE, NHIF, NSSF mandatory. | Monthly PAYE/NHIF/NSSF remittance to KRA/NHIF/NSSF by 9th of each following month. Non-compliance: 5% penalty + interest. |
| Kenya Revenue Authority — Cooperative Surplus Tax | 30% on taxable annual surplus above KES 10M exempt tranche. | At KES 98.6M taxable surplus (gross surplus KES 108.6M − KES 10M exempt): annual tax KES 29.6M. Net cooperative surplus ≈ KES 79.0M/year. Full tax compliance is essential for a cooperative receiving government AHP support. |
| Stamp Duty Act | 2% on property transfer value (peri-urban). Payable by buyer at title deed registration. | 3BR unit: KES 97,350 stamp duty per transaction. Applies to all 2,000 units when title deeds are transferred from developer/cooperative to individual buyers. |

## 15.3 Environment

| REGULATION | REQUIREMENT | ANGAWATCH COMPLIANCE |
| --- | --- | --- |
| NEMA EIA (Environmental Management Act Cap 387) | Class A EIA mandatory for developments over 50 residential units | EIA to be filed with NEMA 6 months before groundbreaking. Timeline: 4–6 months for approval. Budget: KES 5–8M for consultants. |
| NEMA Water Quality Standards | Treated wastewater meets WHO/KEBS irrigation water quality standards before farm application | Reed bed + slow sand filter treatment chain (Section 13.4) meets these standards. |
| Climate Change Act 2016 | GHG reporting for projects receiving public finance | Annual GHG inventory submitted using GPC 1.1 methodology (Global Protocol for Community-Scale GHG Inventories). |

## 15.4 Water

| REGULATION | REQUIREMENT | ANGAWATCH COMPLIANCE |
| --- | --- | --- |
| Water Resources Authority Act (WRA 2016) | Water abstraction permit for Yatta Canal irrigation off-take | Application to WRA Machakos sub-office. Yatta Canal abstraction for irrigation has historical precedent (existing Yatta scheme farmers). Expected approval 3–6 months. |
| Water Act 2016 | Water quality standards for any non-potable water reuse | Greywater recycled for toilet flushing meets secondary treatment standard. Not used for potable purposes. |

## 15.5 Transport and Telecommunications

| REGULATION | REQUIREMENT | ANGAWATCH COMPLIANCE |
| --- | --- | --- |
| NTSA Electric Vehicle Regulations 2023 | Registration of EV fleet. Roam vehicles pre-certified NTSA. | 5 Roam Rapid + 50 Roam Air — all NTSA type-approved. Fleet registered as community transport cooperative. |
| Communications Authority of Kenya | Frequency allocation for IoT smart metering. Safaricom USSD shortcode registration. | USSD shortcode *384# (or similar) registered with CA. Daraja API integration with Safaricom standard commercial agreement. |

SECTION 16 FINANCIAL MODEL — CORRECTED AND COMPLETE

## 16.1 Total Project Cost — Revised

| COMPONENT | KES (MILLION) | NOTES |
| --- | --- | --- |
| Land (102ha / 252 acres @ KES 1.1M/acre) | 277 | 252 acres × KES 1.1M = KES 277.2M. Updated from 246M (90ha/225 acres) — site expanded to 102ha for G+2 maisonette terraces with private gardens. |
| Residential Construction (159,850 m² × KES 46,500/m² VAT-inclusive) | 7,433 | Pure construction cost only: 150×42m² + 1,130×75m² + 700×95m² + 20×115m² = 159,850m² × KES 46,500 = KES 7,433M. Consistent with the KES 76,200/m² all-in benchmark in Section 17. |
| Per-unit community infrastructure allocation (2,000 × KES 450K) | 900 | Roads, internal sewers, utility connections, meter cabinets, biogas stub — allocated per unit as community infrastructure. Separate from the raw construction cost above. |
| Solar PV (3,360 kWp @ KES 95/Wp) | 319 | 7,680 × Jinko JKM425N-54HL4R-BDB 425W panels + 40 × SMA/Huawei 100kW inverters. Sunny Design validated 5,480 MWh/year yield. |
| BESS — 3,600 kWh CATL EnerOne (18 units) | 100 | USD 207/kWh all-in installed |
| Biogas system (6 digesters + H2S scrubber + Cummins genset + HDPE pipeline + meters) | 65 |  |
| Community Health Centre (50-bed) | 260 |  |
| Primary School (20 classrooms) | 130 |  |
| Secondary School (30 classrooms + labs) | 195 |  |
| Worship Centre (interdenominational 800-seat) | 45 | New addition |
| Recreation (football pitch + 20 playgrounds + courts) | 35 | New addition |
| YattaFarm + Cold Storage + Irrigation + Chrysanthemum | 182 |  |
| E-Mobility Fleet (5 Roam Rapid + 50 Roam Air + swap station + carport solar) | 208 |  |
| Water, Waste &** Sanitation (sewage treatment + stormwater + cisterns)** | 338 |  |
| Underground XLPE Cabling + Smart Meters (2,000 × Kaifa MA105) | 220 |  |
| KPLC Grid Interconnect (4km overhead 33kV to Matuu substation) | 36 |  |
| Community Hub, Market, Green Spaces, Sports | 260 |  |
| M-Pesa billing software + IT infrastructure | 2.5 |  |
| NEMA EIA + WRA permits + EPRA licences (professional fees) | 15 |  |
| TOTAL PROJECT COST | 12,212 | ~$94M at KES 130/USD. Land 277M + residential construction 7,433M + per-unit infrastructure 900M + all other components 3,602M = KES 12,212M. Updated from 12,181M — land cost corrected for 102ha site. |

## 16.1b Tax Framework — What the Document Has Not Previously Stated

Every Kenyan financial analysis must account for applicable taxes. The following taxes materially affect both household affordability and community cooperative revenue. Their omission in previous versions was a significant gap.

| TAX / DUTY | RATE | IMPACT ON ANGAWATCH | NOTES |
| --- | --- | --- | --- |
| PAYE (Income Tax) on resident salaries | Progressive: 10% / 25% / 30% bands. Personal relief KES 2,400/month. | Reduces combined take-home from KES 110,000 gross to ~KES 85,880 net per household (before AHL). WITH AHL (1.5% × KES 55,000 each = KES 825/earner): combined net ≈ KES 84,230/month. All affordability ratios should use KES 84,230 net for real budgeting. | KMRC formally assesses affordability on gross income — so mortgages qualify at gross. Real household budget runs on KES 84,230 net after PAYE + NHIF + NSSF + AHL. |
| NHIF (National Hospital Insurance Fund) | KES 1,700/month per employee (2024 rates) | Reduces take-home by KES 1,700/month per earner = KES 3,400/month per household. | NHIF also entitles residents to subsidised care at the community CHC — partial offset. |
| Affordable Housing Levy (AHL) | 1.5% of gross salary — employee deduction. MATCHED by employer at 1.5%. | At KES 55,000 gross/earner: AHL = KES 825/month/earner = KES 1,650/month per household. Employer match: cooperative or TSC/MoH pays additional KES 1,650/month per employee — real additional cost to the employment system. | Finance Act 2023 (AHL) — mandatory for ALL employees from Jan 2024. AHL funds go to the Affordable Housing Fund. Ironically, Angawatch residents pay into the same fund that subsidises their purchase via AHP — a circular benefit. |
| NSSF (National Social Security Fund) | Tier I (KES 420) + Tier II (salary-based) ≈ KES 2,160/month per earner | Reduces take-home by ~KES 4,320/month per household. | NSSF builds retirement savings — deferred income, not lost income. |
| Stamp Duty on property purchase | 2% of property value (rural/peri-urban classification — Kithimani ward) | 3BR (KES 4,867,500 VAT-adjusted): stamp duty = KES 97,350. Plus legal fees KES 20,000 + KMRC registration KES 4,868. | One-off cost at title transfer. Combined with 10% deposit (KES 486,750) = KES 608,968 total cash at purchase. See Section 16.1d. |
| Annual County Land Rates | Set by Machakos County — estimated KES 2,000–4,000/unit/year | KES 167–333/month additional household cost not in previous affordability table. | Community cooperative pays the rates on cooperative-owned rental units. |
| VAT on construction materials | 16% on most building materials. Solar panels, batteries, CSEB: EXEMPT under Finance Act 2023. | Blended effective rate ≈ 8% on total construction cost (exempt items reduce overall VAT burden). | Moves construction cost from KES 43,000/m² to ~KES 46,500/m² blended (VAT-inclusive). |
| Cooperative Surplus Tax | 30% on taxable surplus above KES 10M exempt tranche (Income Tax Act Cap 470) | On KES 98.6M taxable surplus (gross KES 108.6M − KES 10M exempt): KES 29.6M tax payable annually. Net cooperative surplus ≈ KES 79.0M/year. | Cooperative registered under Cap 490. Annual tax compliance required. |
| VAT on rental income | 16% on commercial rental income (not applicable to residential) | Commercial ground-floor leases (KES 34M/year) attract 16% VAT collected from tenants. Cooperative remits to KRA. | Net to cooperative: same KES 34M — VAT is charged to tenants and passed through. |
| Import duty on BESS, solar, EVs | 0% import duty on renewable energy equipment + EVs. VAT-exempt. | No material tax impact on energy system capital costs. Correctly reflected in current costings. | Finance Act 2023, Schedule 2 — explicitly covers solar panels, batteries, EV components. |

## 16.1c Revised Household Affordability — Gross vs Net Income

IMPORTANT CORRECTION: All previous affordability tables used gross income. The corrected analysis separates gross income (used for KMRC mortgage qualification) from net income (used for actual household budgeting).

| COMBINED INCOME BASIS | 3BR EMI (VAT-ADJUSTED) | % OF GROSS | % OF NET TAKE-HOME | VERDICT |
| --- | --- | --- | --- | --- |
| KES 110,000 gross (mid-level — most residents) | KES 40,803 | 37.1% gross | 48.4% net (est. KES 84,230 net after PAYE+NHIF+NSSF+AHL) | Exceeds KMRC guideline. On net: very tight. AHP subsidy non-negotiable. |
| KES 127,000 gross (experienced professionals) | KES 40,803 | 32.1% gross | 41.8% net (est. KES 97,600 net after all deductions incl. AHL) | Within KMRC 30% guideline on gross. Manageable with dual income and AHP. |
| KES 134,000 gross (senior grades) | KES 40,803 | 30.5% gross | 39.4% net (est. KES 103,500 net after all deductions incl. AHL) | Just within KMRC guideline on gross. Dual income provides resilience at this grade. |

WHY 4BR IS ONLY 20 UNITS: At KES 48,615/month EMI (VAT-adjusted), the 4BR requires KES 162,000+ combined gross. After PAYE + NHIF + NSSF + AHL, net income at this level is ~KES 118,000/month. The EMI of KES 48,615 is 41% of net — still very tight. The realistic senior pool at Angawatch is 50–70 households, but most will choose the lower-risk 3BR over the 4BR stretch. 20 units is the honest demand estimate. Unsold units become cooperative premium rentals at KES 42,000–45,000/month.

KMRC‘s affordability guideline (30% of gross income) is the formal lending standard — assessed on GROSS income before all deductions. In reality, a household earning KES 110,000 gross takes home only KES 84,230 net (after PAYE + NHIF + NSSF + AHL at 1.5%). The 3BR at KES 40,803 EMI is 37.1% of gross (borderline) and 48.4% of net (genuinely tight). NOTE ON RATE: All EMIs include KMRC’s mandatory Mortgage Reducing Insurance Scheme (MRIS) premium of ~1.5% p.a., giving an effective rate of ~10.45% p.a. versus the 8.99% headline rate. MRIS is a standard KMRC requirement on all loans. AHP subsidy is not optional — it must reduce effective EMI to under 35% of net income for this unit to be viable for mid-level earners. The 2BR at KES 33,015 (39.2% of net KES 84,230) is still tight but more manageable.

## 16.1d Cash Required at Purchase — 3BR Unit

| COST ITEM | AMOUNT | NOTES |
| --- | --- | --- |
| 10% deposit (KMRC requirement) | KES 486,750 | 10% of KES 4,867,500 (VAT-adjusted unit cost) |
| Stamp duty (2% — peri-urban classification) | KES 97,350 | Paid to KRA at title transfer |
| Conveyancing lawyer fees | KES 20,000 | Standard Machakos County rate |
| KMRC loan registration fee | KES 4,868 | 0.1% of loan amount |
| TOTAL CASH REQUIRED AT PURCHASE | KES 608,968 | A nurse-teacher household saving KES 15,000/month takes ~3.4 years to accumulate this. The cooperative rental tier bridges this gap. |

## 16.2 Household Affordability — 3BR Primary Target Family

| MONTHLY EXPENSE | ANGAWATCH (KES) | NAIROBI SATELLITE (KES) | MONTHLY SAVING (KES) |
| --- | --- | --- | --- |
| Housing (3BR maisonette EMI — VAT-adjusted @ KMRC 8.99% + ~1.5% MRIS = effective 10.45% over 25yr) | 40,803 | 55,000 | 14,197 |
| Electricity (KES 9/kWh community rate, 150 kWh/month) | 1,350 | 11,500 | 10,150 |
| Cooking fuel (community SACCO LPG — bulk procurement discount) | 1,295 | 1,560 (retail LPG) | 265 |
| Water (reduced by recycling + harvesting) | 800 | 3,000 | 2,200 |
| Transport (e-matatu Angawatch–Machakos) | 2,200 | 7,040 | 4,840 |
| School fees (community school, affordable) | 2,000 | 8,000 | 6,000 |
| County land rates (Machakos County annual charge) | 250 | — | −250 |
| Net metering credit (solar excess → M-Pesa) | −500 | 0 | 500 |
| TOTAL MONTHLY EXPENDITURE | 44,107 GROSS basis | 86,100 | 42,493 SAVED (gross basis). On NET income: household pays KES 44,107 from ~KES 85,880 net = 51.4% of net income. This is why dual income and AHP subsidy are both essential for 3BR purchasers. |
| % of combined KES 110,000 GROSS income | 39.8% gross / 51.0% net | 78.3% | 38.5 ppts gross saved. On net take-home (KES 85,880): household spends 51.0% on this list — dual income resilience and AHP subsidy both essential. |
| % of KES 134,000 (senior grades) | 32.4% — near ceiling | 64.3% | Within manageable range for established professionals |

Corrected total monthly saving: KES 34,943 on housing + electricity + cooking (SACCO LPG) + transport + water + net metering. With community school fee savings: ~KES 40,943/month. The cooking saving is now KES 265/month (SACCO LPG vs retail LPG) not KES 810 — the biogas-to-homes claim was removed as infeasible.

## 16.3 Community Revenue — Year 5+ (Revised)

| REVENUE STREAM | ANNUAL (KES M) | NOTES |
| --- | --- | --- |
| Grid export (1,379 MWh @ KES 12/kWh) | 16.5 | EPRA net metering. 5,734 − 4,355 = 1,379 MWh × KES 12 = KES 16.5M/year. Solar yield Sunny Design validated. |
| Community enterprise + market leases | 58.0 | Jua Kali + shops + offices |
| Commercial ground-floor apartment leases | 34.0 | Ground floors commercial all blocks |
| YattaFarm food crops | 22.0 | Community + Nairobi market |
| Chrysanthemum plot (4ha @ KES 850K/ha) | 3.4 | Domestic cut flower market. New addition. |
| Digestate — fertiliser import substitution (cost saving, not cash revenue) | 0 cash / KES 14.5M OPEX saving | ACCOUNTING CORRECTION: The digestate is applied to the 20ha YattaFarm and 5ha Napier grass fodder plot as organic fertiliser. It cannot simultaneously be sold to external farms. The financial benefit is real (KES 14.5M/year in avoided chemical fertiliser purchases), but it is correctly classified as an OPEX reduction, not cash revenue. See operating costs row. |
| E-matatu transport cooperative (5 buses) | 13.1 | KES 80/passenger × 14 seats × 80% occupancy × 8 runs × 5 buses × 365 |
| 1BR cooperative rental income (150 units × KES 11,000 × 12) | 19.8 | ONLY the 150 × 1BR studios are retained as cooperative rental stock. Generates safe, manageable KES 19.8M/year with no billion-shilling debt burden. 1,130 × 2BR and all maisonettes are SOLD. |
| BESS grid ancillary services | 7.0 | Frequency regulation contract with KPLC |
| CSEB block enterprise external sales | 4.8 | Post-construction at KES 45/block |
| CHC health contracts (ACHIH + county) | 3.5 | Telemedicine + county health data |
| TOTAL CASH REVENUE (Year 5+) — GROSS | 182.1 | ~$1.40M/year. 1BR rental 19.8 + grid export 16.5 + enterprise/commercial 92.0 + YattaFarm CASH 25.4 (food 22.0 + chrysanthemum 3.4 only — biochar and digestate both retained on-site) + e-matatu 13.1 + BESS 7.0 + CSEB 4.8 + CHC 3.5 = KES 182.1M. Biochar sales row removed: biochar from digestate solid fraction is pyrolised and applied to YattaFarm soil — cannot simultaneously be sold externally while retaining all digestate for farm fertility. |
| Less: Total operating costs | −73.5 | Adjusted: previous KES 88M OPEX minus KES 14.5M fertiliser cost (now eliminated by digestate import substitution). Staff salaries, O&M, admin, maintenance: −82.0M. Less: digestate fertiliser import substitution saving: +14.5M. Net OPEX: −73.5M. |
| Less: Cooperative surplus tax (30% on KES 98.6M taxable) | −29.6 | Income Tax Act Cap 470. Taxable base: KES 108.6M gross surplus − KES 10M exempt tranche = KES 98.6M × 30% = KES 29.6M. |
| 1BR property operating costs (KES 19.8M rental stock) | −6.0 | Maintenance, insurance, management — small portfolio, low cost |
| All other operating costs (farm, EV, CHC, admin, cooperative) | −82.0 | As previously estimated |
| Cooperative gross surplus before tax | ~108.6 | Cash revenue 182.1 − adjusted OPEX 73.5 = 108.6M. Reduction from 114.6M: biochar sales (KES 5.5M) removed — cannot sell external while applying all digestate to own farm. |
| Cooperative surplus tax (30% on taxable above KES 10M exempt) | −29.6 | 109.1 − 10 = 99.1 × 30% |
| NET COOPERATIVE SURPLUS (after all costs + tax) | ~79.0 | ~$0.61M/year. 40% resident dividend (KES 31.8M), 30% maintenance (KES 23.8M), 20% expansion (KES 15.9M), 10% county (KES 7.9M). Honest closed-loop figure: all organic outputs retained on-site. |

## 16.4 Financing Structure — Two-Phase Capital Stack

IMPORTANT REAL ESTATE FINANCE CLARIFICATION: This section separates CONSTRUCTION FINANCE (how the buildings are funded during the 4-year build) from EXIT/TAKE-OUT FINANCE (how the construction debt is repaid once units are sold). These are two distinct phases. KMRC mortgages are NOT construction finance — they are the take-out mechanism. The developer builds using DFI and equity capital, sells completed units to families who take KMRC mortgages, and uses those sales proceeds to repay the DFI construction bond. Listing KMRC and DFI in the same column as co-equal sources would be a capital-stack double-count — they represent the same money at two different moments in time.

PHASE 1 — Construction Capital Stack (how we build it): KES 12,212M

| INSTRUMENT | KES M | % | NOTES |
| --- | --- | --- | --- |
| DFI Concessional Green Bond (AfDB/IFC/AIIB) | 8,037 | 65.8% | Primary construction lender. 15-year tenor, ~4–5% interest. Increased by KES 31M to cover corrected land cost (102ha vs 90ha). Repaid from KMRC sales proceeds at project completion — see Phase 2. |
| Kenya AHP + State Department of Housing Fund | 1,680 | 13.8% | Affordable Housing Programme grant/subsidy + Housing Fund concessional debt. Covers the AHP-eligible units (3BR at 37% of gross income). Machakos County CIDP 2023–2027 explicitly allocates housing development funds to peri-urban satellite developments. |
| Machakos County Infrastructure Bond | 1,000 | 8.2% | 10-year county bond for public infrastructure: roads, potable water reticulation, schools, CHC. Serviced by county revenue, not cooperative income. |
| Community SACCO + Off-Plan Buyer Deposits | 800 | 6.6% | 10% KMRC deposits collected off-plan from committed buyers. Faulu SACCO / KWFT facilitate. Reduces net DFI drawdown. |
| Verra VCS Carbon Credits (forward sale) | 163 | 1.3% | 22,910 tonnes CO₂/yr × 25yr × $6/tonne upfront. Verified Carbon Standard project registration. |
| Developer Equity + EPC Contractor Co-investment | 532 | 4.4% | First-loss equity. Developer earns carried interest from Year 10. EPC contractor takes equity stake in lieu of part-fee. |
| TOTAL CONSTRUCTION CAPITAL | 12,212 | 100% | Sum: 7,974+remaining sources. KMRC 7,974M confirmed (4,449+3,409+116). Construction stack total 12,212M. |

PHASE 2 — Exit / Take-Out Finance (how we repay construction debt)

| EXIT MECHANISM | KES M | TIMING | HOW IT WORKS |
| --- | --- | --- | --- |
| KMRC Sectional Title Mortgages — 1,130×2BR + 700×3BR + 20×4BR | 7,974 | Years 3–5 as units complete | 2BR: 1,130×4,449M. 3BR: 700×3,409M. 4BR: 20×116M. Sum: 4,449+3,409+116 = 7,974M exactly. Proceeds retire the DFI construction bond. This is take-out finance, not construction capital. |
| 150×1BR cooperative rental stock — retained long-term | ~KES 180M book value | Held permanently | These 150 units are NOT sold. The cooperative retains them as rental stock (KES 19.8M/year income). A small residual DFI balance (~KES 33M) covers this retained stock — a manageable long-term cooperative liability. |
| NET DEBT POSITION AT PROJECT COMPLETION | ~KES 64M remaining | By end Year 5 | After KMRC sales retire the DFI bond: DFI balance = 8,037 − 7,974 = KES 63M residual. This small balance covers the 150 cooperative rental apartments. All other construction debt is fully retired by sales proceeds. |

## 16.5 20-Year Cash Flow Projection — Corrected

CORRECTION FROM PREVIOUS VERSION: Earlier cash flow showed only KES 4.1B CAPEX against a KES 12.2B project cost — a KES 8B gap that would immediately disqualify the proposal from DFI review. The corrected model shows: (1) full CAPEX of KES 12,212M distributed across build years, (2) unit sales proceeds (~KES 7,974M from 1,130 × 2BR + 700 × 3BR + 20 × 4BR — corrected from 7,973M) as a separate income stream that offsets construction debt, and (3) cash operating revenue at KES 182.1M/year (biochar retained on-site — not sold; digestate as OPEX saving). Net surplus: KES 79.0M/year.

| YEAR | CAPEX (KES M) | OPEX (KES M) | OPERATING REVENUE | UNIT SALES PROCEEDS | NET (KES M) | CUMULATIVE (KES M) |
| --- | --- | --- | --- | --- | --- | --- |
| Year 1 — Phase 1 (Site + Permits + Biogas Civil) | –750 | –15 | 0 | 0 | –765 | –765 |
| Year 2 — Phase 2 (Energy + Schools + CHC + Water) | –3,400 | –30 | 0 | 0 | –3,430 | –4,195 |
| Year 3 — Phase 3 (75% Residential + Farm + Amenities) | –5,300 | –55 | +95 | 3,189 | –2,071 | –6,071 |
| Year 4 — Phase 4 (Complete Build + E-Mobility + Enterprise) | –2,731 | –75 | +189 | 3,189 | +572 | –5,499 |
| Year 5 — Full operation (remaining sales complete) | 0 | –74 | +183 | 1,594 | +1,703 | –3,796 |
| Year 7 | 0 | –82 | +202 | 0 | +120 | –3,670 |
| Year 10 — BESS partial maintenance (cell balancing, BMS) | –80 | –110 | +240 | 0 | +50 | –3,620 |
| Year 15 | 0 | –140 | +284 | 0 | +144 | –4,160 |
| Year 17 — BESS full replacement (6,000-cycle rated life) | –100 | –155 | +300 | 0 | –55 | –4,215 |
| Year 18 | 0 | –160 | +316 | 0 | +156 | –4,004 |
| Year 20 | 0 | –175 | +344 | 0 | +169 | –3,835 |
| Year 25 — Solar panel refresh | –380 | –195 | +404 | 0 | –171 | ~breakeven from Year 24 |

TOTAL BUILD CAPEX: KES 12,212M (Years 1–4) + KES 80M BESS partial maintenance (Year 10) + KES 100M BESS full replacement (Year 17) + KES 380M solar refresh (Year 25) = KES 12,772M lifecycle capital. UNIT SALES (Years 3–5): KES 7,974M total from 1,130 × 2BR (KES 4,449M) + 700 × 3BR (KES 3,409M) + 20 × 4BR (KES 116M). Corrected from 7,973M — addition check: 4,449+3,409+116 = 7,974M exactly. Project equity IRR: ~12–15% (including Year 17 KES 100M BESS full replacement). Social IRR (health + education): ~22–28%. The cumulative deficit narrows as operating revenue grows and the 3BR cooperative rentals generate recurring income.

SECTION 17 BENCHMARKS — ANGAWATCH VS. YATTA SUB-COUNTY AND MACHAKOS COUNTY

All benchmarks follow the GPC 1.1 methodology (Global Protocol for Community-Scale Greenhouse Gas Inventories v1.1, ICLEI/WRI/C40, 2021). Baseline figures from Machakos County CIDP 2023–2027, KNBS 2019 Census, and KPLC published data. Comparison population: Machakos County (urban+rural) and Yatta sub-county specifically.

## 17.1 Housing Affordability

| METRIC | CURRENT MACHAKOS/YATTA | ANGAWATCH | IMPROVEMENT |
| --- | --- | --- | --- |
| All-in capital cost per m² (total built area) | N/A — informal settlements | KES 76,394/m² (KES 12,212M ÷ 159,850m² total built residential area) | CALCULATION: 1BR×150×42m²+2BR×1,130×75m²+3BR×700×95m²+4BR×20×115m²=159,850m² total built area. KES 76,200/m² is 10–28% BELOW standard Nairobi new-build at KES 85,000–120,000/m². Previous metric divided by land area (90,000m²) — incorrect. Cost/m² should always use built floor area, not plot area. |
| Monthly housing cost as % of income (3BR) | KES 14,000–20,000 rental (no equity, no security) | EMI KES 38,012 = 28.4–40% depending on income grade (full ownership, title deed) | Renting forever → owning in 25 years. With AHP subsidy: reduces to 24–30%. |
| Housing security | Month-to-month rental — eviction risk | 25-year mortgage → full title deed | Lifelong security vs perpetual rental |
| Energy costs as % of income | KES 6,000–10,000/month KPLC = 4.3–7.1% | KES 1,350/month = 0.96–1.1% | 64–87% reduction |

## 17.2 Interior Air Quality — Critical WHO Benchmarks

| POLLUTANT | CURRENT YATTA (CHARCOAL) | ANGAWATCH PHASE 1 (LPG HOME) / PHASE 2 (BIOGAS INSTITUTIONS) | WHO SAFE LIMIT | IMPROVEMENT |
| --- | --- | --- | --- | --- |
| PM2.5 during cooking | 300–1,000 µg/m³ (charcoal) | 10–30 µg/m³ Phase 1 (LPG — 97% below charcoal, still above WHO annual guideline but far safer). < 10 µg/m³ Phase 2 (biogas at CHC/schools). ~0 µg/m³ Phase 3 (solar induction) | 15 µg/m³ (annual) / 45 µg/m³ (24-hr) | Phase 1: 97–99% reduction vs charcoal. Phase 3: 100% elimination. |
| Carbon monoxide (CO) | 50–200 ppm | < 5 ppm | 10 ppm (8-hour) | 95%+ reduction |
| NOx during cooking | 20–40 ppm | < 5 ppm | No WHO specific limit — lower is better | 75–87% reduction |
| Annual PM2.5 per household | 6.57 kg/year (charcoal baseline) | ~0.05 kg/year Phase 1 (LPG) → ~0 Phase 3 (solar induction) | — | Phase 1: 99.2% reduction. Phase 3: 100%. |
| Community PM2.5 eliminated from charcoal baseline | — | ~13.0 tonnes/year (LPG emits ~0.05 kg/year vs charcoal 6.57 kg/year) | — | Equivalent to removing 2,575 diesel cars from road |

## 17.3 Energy Availability and Reliability

| METRIC | CURRENT YATTA/KPLC | ANGAWATCH | IMPROVEMENT |
| --- | --- | --- | --- |
| Annual power availability | ~82% (KPLC reported 97.4% nationally but rural Yatta significantly lower with 15–25 outages/year) | ~99.6% | 17.6 percentage point improvement |
| Power outages per year | 15–25 incidents/year averaging 4–8 hours each | < 2 incidents/year (grid events only; community microgrid self-heals) | 85–92% reduction in outage events |
| Average outage duration | 4–8 hours | < 30 minutes (automatic BESS transfer) | 95% shorter when outages occur |
| Energy cost during outage | Resident pays for candles/generator fuel | Zero — BESS maintains supply | Full saving |

## 17.4 Carbon Intensity

| METRIC | CURRENT MACHAKOS | ANGAWATCH | NOTES |
| --- | --- | --- | --- |
| Household annual electricity emissions (Scope 2) | KPLC grid: 0.58 kgCO₂/kWh × 1,236 kWh/year (= 103 kWh/month × 12, per Angawatch 2BR appliance audit) = 0.717 tonnes CO₂/year | ~0 (98% renewable generation) | Carbon intensity ≈ 0.01 kgCO₂/kWh. NOTE: Previous figure of 3,000 kWh/year was inconsistent with our own appliance-level energy audit (103 kWh/month). Corrected to be internally consistent. |
| Household cooking emissions (Scope 1) | Charcoal: 3.02 kgCO₂/kg × 550 kg/year = 1.66 tonnes CO₂/year | LPG: 3.0 kgCO₂/kg × 96 kg/year = 0.288 tonnes CO₂/year. CHC/schools: biogas = 0 Scope 1 (biogenic, GPC 1.1) | 83% reduction in cooking Scope 1 emissions (charcoal → LPG). Full elimination by Phase 3 solar induction. |
| Total household annual emissions | ~2.37 tonnes CO₂/year (0.717 KPLC + 1.66 charcoal cooking corrected baseline) | ~0.36 tonnes CO₂/year Phase 1 (0.288 LPG + 0.07 KPLC) → ~0.07 Phase 3 (solar induction) | Phase 1: 84.8% reduction. Phase 3: 97.0% reduction. KPLC baseline corrected from 1.74 to 0.717 tonnes using own audit figure of 1,236 kWh/year. |
| Carbon intensity per KES 110,000 income | 21.5 gCO₂/KES of income (2.37 tonnes / KES 110,000) | 3.27 gCO₂/KES Phase 1 → 0.64 gCO₂/KES Phase 3 | Phase 1: 84.8% reduction. Phase 3: 97.0% reduction. |
| Community annual carbon saving | — | 22,910 tonnes CO₂/year equivalent | At Verra VCS $6/tonne: KES 17.9M/year |
| PM2.5 eliminated | — | 13.14 tonnes PM2.5/year | WHO: equivalent to 2,600 diesel cars removed |

SECTION 18 PHASED IMPLEMENTATION — 4 YEARS TO FIRST OCCUPANCY

| PHASE | TIMELINE | ACTIVITIES | COST (KES M) | MILESTONE |
| --- | --- | --- | --- | --- |
| Phase 1 — Permits &** Foundations** | Months 1–12 | Land acquisition, NEMA EIA, WRA permit, EPRA DG+gas licences, KPLC grid agreement, site clearing, infrastructure backbone (roads, drainage, underground cables, biogas plant civil works) | 580 | NEMA EIA approved. All licences granted. Site ready. |
| Phase 2 — Energy Infrastructure + Schools | Months 7–24 | Solar PV installation (rooftops erected as buildings complete). CATL BESS installation in 4 energy hubs. Biogas digesters commissioned. Primary + secondary schools built. CHC built and equipped. | 2,100 | Community energy system live. Schools and CHC operational. |
| Phase 3 — Residential + Amenities | Months 13–42 | 2,000 apartments constructed (all 4 types). Ekeja clusters completed with courtyards, laundry courts, and playgrounds. Worship centre. Jua Kali zone. YattaFarm irrigation and cold storage. | 5,800 | First 800 families move in Month 18. Full 2,000 families by Month 42. |
| Phase 4 — Enterprise + E-Mobility | Months 36–48 | EV fleet commissioned. Boda swap station. Commercial strip occupied. Community market operational. M-Pesa SmartMeter billing live. CSEB block factory operational. | 870 | Full cooperative revenue from Month 48. |
| TOTAL | 48 months | All phases overlap — energy infrastructure leads housing | ~9,350 (contingency separate) | First family moves in: Month 18. Full community: Month 42. |

SECTION 19 RISK MITIGATION — EYES-OPEN DESIGN

| RISK | LIKELIHOOD | IMPACT | MITIGATION |
| --- | --- | --- | --- |
| EPRA gas distribution licence delay | Medium | High | Pre-application meeting with EPRA in Phase 1 Month 1. EPRA has approved similar networks. Legal precedent exists. |
| Biogas feedstock shortfall (Phase 1–2 before full occupancy) | Medium | Medium | Supplementary feedstock from YattaFarm organic waste + Matuu market waste collection agreement. Phase 1 digesters run at 40% capacity — acceptable. |
| KPLC net metering agreement delay | Medium | Low | Community financially viable without export revenue. Grid export is bonus income, not a revenue dependency. |
| KMRC mortgage rate increase above 8.99% | Medium | Medium | Project viable at 10% rate (3BR EMI rises to ~KES 37,000 = 30.8% of KES 120,000). AHP subsidy provides partial rate protection. |
| Solar panel supply chain disruption | Low | Medium | Jinko Solar has Nairobi distributor (Solar World Kenya) with 12-month forward stock. Alternative: LONGi panels from same distributor. |
| WRA Yatta Canal water permit | Low | Low | Canal abstraction for irrigation has 60-year historical precedent. WRA (Water Resources Authority — renamed from WRMA under Water Act 2016). Community volume small (drip for 20ha). Permit expected within 3–6 months. |
| Biogas H2S scrubber failure | Low | High | Iron sponge media requires replacement every 6 months. Maintenance protocol written into community management agreement. Spare media always on site. |
| Land acquisition dispute | Medium | High | Community land trust structure. County government partnership. Phase 1 allows 12 months for legal resolution before construction begins. |
| COMMON CAUSE FAILURE 1 — Prolonged cloud April + biogas feedstock drop simultaneously | Low | High | April rains actually INCREASE organic feedstock (more YattaFarm waste). This double-adverse scenario is self-limiting. KPLC grid connection is the final backstop. |
| COMMON CAUSE FAILURE 2 — Institutional biogas pipeline rupture (CHC/schools/market) | Low | Medium | Pressure loss sensor on each institutional building inlet triggers automatic solenoid shut-off at building entry point. Isolates affected building without disrupting other buildings. Standby LPG cylinder at CHC ensures no interruption to critical cooking/sterilisation. |
| COMMON CAUSE FAILURE 3 — BESS thermal event at Energy Hub | Very Low | High | CATL LFP chemistry: thermal runaway threshold 270°C vs 210°C for NMC. LFP chosen specifically for this risk. Each Energy Hub is a separate containerised unit with independent BMS and fire suppression system. No single hub contains more than 5 of the 18 units. |

SECTION 20 FEASIBILITY ASSESSMENT — EVERY COMPONENT VERIFIED

| COMPONENT | ASSESSMENT | VERIFIED BASIS |
| --- | --- | --- |
| Solar PV (3,360 kWp effective) | FEASIBLE — INDEPENDENTLY VALIDATED | SMA Sunny Design 6.11.2 confirms 5,479.85 MWh/year yield. Jinko JKM425N-54HL4R-BDB panels commercially available in Kenya. 40 × SMA SHP 100-20 400V inverters (or Huawei equivalent). Max DC voltage 973V confirmed safe. PR 0.82 validated. |
| Institutional biogas (CHC, schools, market) + electricity | FEASIBLE | 773 m³/day from feedstock (8,090 residents + 40-cow commercial dairy lease). Supplies 5 institutional kitchens + 254 MWh/year electricity. Cummins C220N5 biogas-modified units operational at JKUAT, Nakuru. |
| BESS (3,600 kWh LFP) | FEASIBLE | CATL EnerOne units available via African distributors. USD 200–215/kWh all-in installed is 2026 market rate. |
| KMRC Mortgage Financing | FEASIBLE | KMRC rate 8.99% Q1 2026. NCBA, KCB, Co-op Bank. 3BR EMI KES 38,012 at 34.6% of KES 110K income — tight but achievable for experienced-grade staff. AHP subsidy available. |
| Yatta Canal Solar-Pumped Fill to 15,000m³ Pan | FEASIBLE — verified | Canal carries 3 m³/sec (259,200 m³/day). Farm needs 30,000 m³/year = 0.04% of annual canal flow. 10kW solar high-lift pump fills upslope pond during wet seasons (April–May, Nov–Dec). WRA water abstraction permit applied for Phase 1. National Irrigation Board operates pump-fill schemes at Iviani and Kondo villages, Kithimani ward. |
| Biogas to institutions (CHC, schools, market) | FEASIBLE | 80 m³/day for institutional supply from 621 m³/day total production. Pipeline to 5 institutional buildings only. Individual homes on SACCO LPG. |
| Community LPG SACCO (household cooking Phase 1) | FEASIBLE | TOTAL Energies Kenya supplies institutional bulk LPG contracts already in Machakos County. SACCO structure for collective purchasing is standard in Kenya. |
| Solar induction cooking (Phase 3, Year 10+) | FEASIBLE — planned | 876 MWh/year additional load absorbed by growing solar surplus. Single induction hob: 1.2 kWh/day. Standard induction hob KES 3,500–8,000 retail. |
| M-Pesa SmartMeter integration | FEASIBLE — technology exists | Safaricom Daraja API is publicly documented. Kaifa MA105 meter is standard KPLC prepay hardware. STS token engine is open-source. |
| Roam Rapid + Roam Air fleet | FEASIBLE | Both vehicles NTSA type-approved. Roam Motors manufactures in Nairobi Industrial Area. In production Q1 2026. |
| Yatta Canal for power generation | NOT FEASIBLE — REMOVED | Confirmed: canal flow 1.2–2.5 m³/s dry season, gradient too flat. Maximum achievable power: 1–5 kW. Not 120 kW. Canal used for irrigation only. |

SECTION 21 NATIONAL REPLICATION — ANGAWATCH AS A 47-COUNTY TEMPLATE

Angawatch’s core template — solar PV + institutional biogas + community LPG SACCO + BESS + M-Pesa SmartMeter — is directly replicable across all 47 Kenyan counties. The template does not depend on a river, canal, or specific geology. Solar irradiance is sufficient across all counties. Organic waste from any community of 2,000 families supports institutional biogas. LPG SACCO models are already operating in Kenya. The template scales up or down based on community size.

| COUNTY TYPE | ADAPTATION NEEDED | ENERGY MIX CHANGE | EXAMPLES |
| --- | --- | --- | --- |
| Semi-arid plateau (like Yatta) | None — direct replication | Solar + biogas: identical | Taita Taveta, Kwale, Tana River |
| Humid highlands (Kiambu, Muranga) | Lower solar irradiance (5.0 kWh/m²/day). CSEB needs waterproofing render. | More BESS, biogas extended hours | Kiambu, Nyeri, Murang’a |
| Lake Basin (Kisumu, Siaya, Homa Bay) | Fish waste + water hyacinth = additional biogas feedstock. Higher rainfall. | Biogas MORE abundant | Kisumu, Siaya, Homa Bay |
| Arid North (Turkana, Marsabit) | Higher solar (6.0–6.5 kWh/m²/day). Less organic waste. Supplement biogas with solar cooking. | Solar dominant. Less biogas. | Turkana, Marsabit, Wajir |
| Coast (Mombasa, Kilifi) | High humidity. CSEB needs lime stabilisation. Strong sea breeze potential. | Solar + biogas. Possible small wind supplement. | Mombasa, Kilifi, Kwale |

National scale impact: 47 counties × 2,000 families each = 94,000 families = 470,000 Kenyans lifted out of energy poverty. Annual PM2.5 reduction: 13.14 tonnes × 47 = 617.6 tonnes — equivalent to removing 123,000 diesel vehicles from Kenyan roads. Carbon saving: 22,910 tonnes × 47 = 1.08 million tonnes CO₂/year.

SECTION 22 SPRINT PLAN — MARCH 31 TO APRIL 14, 2026

| DATE | TASK | DELIVERABLE |
| --- | --- | --- |
| Apr 1–2 | Download PVGIS CSV for 1.17°S, 37.45°E. Confirm monthly irradiance data. Google Earth screenshot + GPS coordinates screenshot. | PVGIS data file + satellite image |
| Apr 1 | Contact EPRA pre-application for Gas Distribution Network Licence. Email info@epra.go.ke with project description. | EPRA correspondence |
| Apr 1 | Contact Roam Motors (roamelectric.com) for fleet quotation: 5 × Roam Rapid + 50 × Roam Air + 1 × swap station. | Quotation letter from Roam |
| Apr 1–3 | Update Angawatch_Complete.docx with ALL 39+ corrections. See corrections log. | Updated Word document |
| Apr 3–6 | Field interviews in Matuu + Yatta Town: Verify KPLC bills (photograph), LPG receipts, actual matatu fares, boda fuel receipts. Film Yatta Canal gravity flow. Record site video. | Interview footage + receipts + field photos |
| Apr 3–5 | Energy calculations Excel: Solar yield (PVGIS-based), biogas mass balance, BESS sizing, 20-year cash flow, BoQ. | 3-tab Excel workbook |
| Apr 5–8 | Architect brief → CAD floor plans: 1BR, 2BR, 3BR, 4BR, CHC, site masterplan 1:2,000. | 6 CAD drawings (PDF) |
| Apr 7–10 | Datawrapper charts: Monthly solar output (bar), energy mix pie, benchmark comparison, PM2.5 before/after, two-season daily profile. | 5 publication-quality charts |
| Apr 8–12 | Pitch video (DaVinci Resolve, 10–12 min, English). Structure: Wanjiku’s morning → site location → community needs → energy system → financial proof → replication. Dual audience: family story + engineering proof. | YouTube unlisted link |
| Apr 10–12 | Slide deck: 74 slides per SlideBreakdown.docx. Embed charts, renders, floor plans. | PowerPoint + PDF |
| Apr 11–13 | Full cross-reference review: every number in slides = Excel = Word = video. Identify any remaining inconsistency. | Consistency log |
| Apr 14 (final day buffer) | Package all deliverables: Word doc, Technical Energy Report, slide deck PDF, video link, Excel model. | Submission package |
| Apr 15 by 23:59 GMT (2:59am EAT Apr 16) | Submit via energymentors.org/participate. NOTE: 23:59 GMT = 2:59am East Africa Time on Apr 16. Do NOT confuse 11:59 GMT (2:59pm) with 23:59 GMT (2:59am). | Confirmation email receipt |

SECTION 23 REFERENCES

## Energy &** Technical**

- PVGIS v5.2 — EU JRC (re.jrc.ec.europa.eu/pvgis) — 1.17°S, 37.45°E irradiance data, 12-month verified CSV

- Jinko Solar JKM425N-54HL4R-BDB Tiger Neo N-Type Datasheet — jinkosolar.com — Panel specifications STC and temperature coefficients (2024 model replacing original JKM415M-54HL4-B specification)

- SMA Sunny Design 6.11.2 — sunnydesignweb.com — Independent PV yield validation report for Angawatch, April 7 2026. Project: Angawatch, Location: Kenya/Kalawa, Validated yield: 5,479.85 MWh/year at 1.07°S 37.43°E (Nairobi met station 71km). Report attached as Appendix A.

- Huawei SUN2000-100KTL-M1 Datasheet — huawei.com/en/solar — Inverter MPPT specifications

- CATL EnerOne Containerised BESS Specification — catl.com — LFP chemistry, cycle life, pricing

- KPLC Tariff Schedule 2025 — kplc.co.ke — KES 25/kWh domestic tariff

- EPRA Electricity Supply Industry Report 2024 — epra.go.ke

- EPRA Net Metering Regulations 2022 — epra.go.ke

- IRENA Renewable Power Generation Costs 2023 — irena.org

- ICE Database v3.0 — Inventory of Carbon & Energy, University of Bath — Embodied carbon CSEB

- IEA Africa Energy Outlook 2023 — iea.org

## Vehicles and Transport

- Roam Motors Kenya — roamelectric.com — Roam Rapid (68 kWh, 14-seat) and Roam Air (1.85 kWh swappable) specifications Q1 2026

- Ampersand Battery Swap Stations Rwanda — ampersand.solar — Swappable battery model for boda bodas

## Housing, Construction &** Materials**

- Kenya National Building Code 2023 — Ministry of Lands and Physical Planning

- CRATerre / BASIN — Compressed Stabilised Earth Block Manual 2nd Ed. — cseb.net

- KMRC Mortgage Rates Q1 2026 — kmrc.co.ke — 8.99% long-term rate

- Kenya Affordable Housing Programme Design Guidelines 2025 — housing.go.ke

## Community Needs and Health

- World Health Organisation — Household Air Pollution Fact Sheet, November 2022 — who.int. ‘3.2 million deaths per year. 2.4 billion people cook on solid fuels.’

- WHO Air Quality Guidelines 2021 — PM2.5 safe limits (15 µg/m³ annual, 45 µg/m³ 24-hr)

- KNBS — Kenya National Housing Survey 2019 / Kenya Population Census 2019 — knbs.or.ke

- Machakos County Integrated Development Plan 2023–2027 — machakos.go.ke

## Water and Environment

- WRA Water Resource Allocation Rules 2024 — wra.go.ke

- Verra Verified Carbon Standard VM0044 — verra.org

- GPC 1.1 — Global Protocol for Community-Scale GHG Inventories v1.1 — ghgprotocol.org

- GHG Protocol Scope 1 & 2 Accounting Standards — ghgprotocol.org

- NEMA Environmental Impact Assessment Guidelines 2024 — nema.go.ke

## Finance, Policy and Competition

- AfDB Green Climate Fund Project Preparation Guidelines 2024 — afdb.org

- IFC Performance Standards on Environmental & Social Sustainability — ifc.org

- Kenya Finance Act 2023 — VAT exemption Schedule 2 (solar panels and accessories)

- Energy Mentors International — Power the Community 2026 Competition Brief — energymentors.org/participate

- Energy Mentors International — Framing Your Efforts Guidance Document — energymentors.org

- SNV Netherlands Development Organisation — Biogas Africa Programme Reports 2022–2024 — snv.org